Ex-post Evaluation of Bus Rapid Transit System in Nagoya city

TAKESHITA Hiroyuki1), SHIMIZU Kazuhiro2) and KATO Hirokazu3)

1) Doctoral Student, Graduate School of Environmental Studies, Nagoya University
Furo-cho, Chikusa-ku, Nagoya 464-8603, Japan
E-mail: htake@urban.env.nagoya-u.ac.jp

2) Fukuroi-City Office
1-1-1, Araya, Fukuroi-city, Shizuoka-prefecture, 437-8666, Japan

3) Associate Professor, Graduate School of Environmental Studies, Nagoya University
Furo-cho, Chikusa-ku, Nagoya 464-8603, Japan
kato@genv.nagoya-u.ac.jp

Abstract

This paper analyzes the mid- and long-term effects of “Key Route Bus System” adopting a median bus lane system in the city of Nagoya, which was one of the initial BRT systems in the world in 1985, to examine adaptability. As a result, the analysis suggests follows: 1) Keeping buses less disturbed by the other vehicles, the higher operation speed and the more punctuality can be proved 2) Compared to the ordinary bus system, this system catches a quite bigger patronage. 3) The real capacity to transport passengers does not decline along corridor. 4) The total travel time for bus passengers does not increase because of shorter in-vehicle time in exclusive lanes. As for this result, it is concluded that to median bus lanes as a type of BRT is effective.

- Introduction

Excessive dependence on car mobility evidently causes various environmental problems and even more severe problems with the worsening congestion on roads. Therefore it is necessary to provide better public transportation infrastructures and services within the urban transport system, which is and sooner or later will be suffering from private automobile dependent transport.

There are many examples of the cities in the developed countries that have developed noteworthy public transport systems even before the rapid motorization progress began. However, in many other cities from the developing world, the road network has been improved earlier than the development of public transport and hence has contributed more to the accelerated motorization during the urbanization. Public transport in such cities is often provided by the buses, although huge car traffic causes a remarkable decrease in punctuality and service speeds of the buses, which is followed by a likely decline of the passenger demand. The construction of railroads, although constitutes a real alternative to private automobile transport, requires both huge investment and construction time and therefore fails to provide short term solutions especially in cases where there is lack of financial resources.,. On the other hand, even after the introduction of the new rail systems, in many cases the bus network was not re-organized accordingly and competition among buses and railroads emerges as they serve along the same corridor although the ideal will be the buses rather serving as feeder routes.

BRT System has been recently favored also in developing countries as a new mode of public transportation, therefore it is no wrong to claim that buses have been gaining more interest., BRT systems operating at higher speeds on the exclusive lanes are often regarded as an alternative to railway systems as they hold the advantages of lower construction costs and greater flexibility in route selection over the railways. In the world of rapid motorization and economic growth that the public transport looses its competitiveness, BRT is accepted as an effective public transport mode in many aspects.

However, review of the relevant work on BRT systems has revealed that a detailed ex-post evaluation of BRT systems in terms of their impacts on road traffic, modal shift and induced demand is still poor. On the purpose of disseminating Japanese know-how and experiences, this study examines the BRT System in Nagoya particularly considering on the efficiency of operation and the passenger demand. Nagoya BRT System so called “Key Route Bus System” (Fig-1) is the first Japanese BRT system, and has been serving the central area of Nagoya since 1985. Although Key Route Bus System proves its advantages of higher speed and capacity as it runs on the central lane, unfortunately could not have found any another applications so far.

<Fig-1>

2. BRT System in Nagoya

FTA (Federal Transit Administration) defines the BRT system as "rapid mode of transportation that can combine the quality of rail transit and the flexibly of buses". And TCPR A-23 project provides a more detailed definition. It briefly explains BRT system as: "BRT is a permanently integrated system of facilities, services, and amenities that collectively improve the speed, reliability, and identity of bus transit". BRT systems have a number of advantageous features like:  improvement of running environment; rapid boarding and alighting, fast and efficient fare collection, use of ITS 
(Intelligence Transport System), and integration with other modes. Many of the BRT systems are operated on the exclusive lanes- physically separated right-of-way allowing the buses keeping their schedules. Famous BRT systems in Curitiba, Bogota, and Seoul have the exclusive lanes in the central part of the road. Notably, merit of the median bus lane comes from eliminating the effects of -street parking and left (or right)-turning cars on the transit system. Another type is to provide separate road systems constructed only for the operation of buses (for example Ottawa, Los Angeles etc.), or separate specific systems necessary to run the specific vehicles for some of the BRT systems so called "Guideway Bus" (Essen, Adelaide, Nagoya -Yutorito Line). Table-1 summarizes some examples of BRT systems which have median bus lane.

<Table-1>

2.1 The city of Nagoya

The city of Nagoya is the third largest populated major region in Japan. The total population in 2006 is 2.22 million over an area of 326 km2. The population is almost the same as in Paris (2.14 million in 2004), but the urban area is three times larger (105km2), proving relatively low population density in the city of Nagoya.

The main characteristic of transportation in Nagoya is the extreme provision of roads, substantially improved after the 2nd world war. The area occupied by roads is 16% of the urban area, a similar figure compared with 16% in central Tokyo, or 19% in Osaka City. But the share of road surface in the central city is approximately 40%, making the city of Nagoya the “the city of wide highways” in Japan. Consequently, low population density and extensive road network with high capacity prompoted the car traffic and the the share of the trips made by private automobiles is as much high as 70% which is higher that those of Tokyo or Osaka, the other two biggest settlements in Japan.

The public transport service in the city is provided by suburban railways, subways and busses (including Key Route Bus and Guideway Bus) and operated by “Transportation Bureau of the city of Nagoya” and “Japan Railways”. The total length of subway is 89.1 km and buses are operated on 700km bus network.

2.2 Key Route Bus System of Nagoya

Nagoya Key Route Bus System has been planned to supplement the subway system and the characteristics have been defined as below:

- Exclusive bus lanes in the middle/ center of the road

- Higher operational speed (25km/h) by introducing bus prior signal system and expanding the bus stop intervals (800-1000m)

- Introduction of new vehicles better than conventional buses

- Minimizing the physical and fee impedances of mode transfers (from and to subway)

Some of the given consistent concepts of earlier planning stage have not yet been installed. Fig-2 shows the proposed routes of the Key Route Bus System of which only the two of routes were selected and are currently in operation.

<Fig-2>

In 1982, the first section of the Key Route Bus No. 1 named as “Toko Line”, was opened with its exclusive lanes as curbside bus lane not in the center because the central part of the road is occupied by pillars of the elevated motorway. The Key Route Bus No.2, called “Shin-dekimachi Line”, was opened three years later in 1985 as the first implication of central lane in Japan. The total construction cost is approximately 2.5 million US$ per one kilometer, which is only one percent of the cost for subway. After the introduction of the median bus lanes, the operation speed of the bus during daytime raised by 37% from 14.6 km/h to 19.9 km/h).  In addition, the system allowed higher frequency of 2 minutes headways in the morning rush hours. Therefore, the Key Route Bus System is now an important rapid transit system serving in the city of Nagoya.

Table-2 gives service level of Key Route Bus No.2 and Fig-3 shows its route. The intervals of bus stops are approximately 600m, as twice as the ordinary bus stops’ in the city. At the moment, bus priority signal has not been installed because the frequency of Key route bus No.2 in the rush hour is shorter than the cycle of signals.

However, the exclusive median bus lane is limited to only one route. The major reason why median bus lanes have not much been implemented is the road traffic law in Japan. For the construction of the Key Route Bus No.2 there has been required many case-specific treatments, which are have not been justified by the existing the law and regulations not allowing the buses to run in the central lanes. Therefore, special to Nagoya BRT system, it has been deregulated to allow the buses registered as “Key Route Buses” use such exclusive lanes.

<Table-2>

<Fig-3>

3. Ex-post evaluation of the median bus lane

There are several studies exploring the effects of BRT systems with median exclusive bus lanes (.for example see Levinson et al., 2002 defining BRT system by its seven components). Yabe et al., (2005) defines performance indices to assess the likely effects of BRT on the whole transport system. Similarly, below (Fig-4) sub-sections explore the impacts of the Nagoya Key Route Bus System as an ex-post evaluation of the first and the only BRT system in Japan.

<Fig-4>

3.1 Improvement in operation speed and punctuality

As previously noted, the operating speed was improved by 37% (up to 19.9 km/h) and has been stabilizing around 19 km/h, which is 1.5 times higher than compared to ordinary buses. Table-3 shows the results of the traffic survey carried out to observe the delays and the extent of deviations from the scheduled time of the bus routes the city. Table-3 compares the median lane and curbside lane exclusive bus systems with the ordinary buses to explore potential of the two types of Key Route Bus System to increase the punctuality. The average delays and the standard deviations from the schedule evidently proves better and improved reliability and punctuality of the median  lane system followed by the ordinary and the buses operating on the curbside type of exclusive lanes have yielded the worst punctuality among the three.

<Table-3>

Fig-5 presents the reasons of the increases in the average operation speed after upgrading the ordinary bus system to median lane exclusive bus system, using the results of the speed survey conducted by the Nagoya City Office. 60 % of the speed improvements are attributed to the reduction in the running time provided by the median bus lane. The second reduction of 25.7% is the result of the decreased number of bus stops (from 23 to 17).

<Fig-5>

Other reasons for the higher speeds are the restrictions of on-street parking, left-turns (Japan has left-side traffic). These effects are not separately reported here, but included in the following factors of time saving: stops at signals (13.3%); and traffic jam (4.0%).

In order to examine the impacts of on-street parking, we used the Traffic Census, 1999 for "rush hours on-the-street parking car statistics" and the parking time model suggested by Nakamura et al., (2004) and estimated 9.19 parked vehicles per kilometer.

In other words, approximately in every 110m one bus passes by a vehicle parked on the street. Besides, the performance of the curbside lane is easily affected by the left turn movement of the vehicles. Therefore we conclude that the worst punctuality is attributable to the traffic and parking condition on the road.

However, the ordinary vehicles can easily intrude into the exclusive median lane since it is not a system of exclusive lane separated physically from the other lanes. We estimated that 1.07 vehicles enter the median bus lane per kilometer at the average with less impact by the left turns since they move in the same direction with the buses.

The average stop time at the signals constitutes the 27％ of the total service time on the road in other words to give a better understanding, is that If the Key Route Bus would not have stopped at signals, the operation speed could have been improved from 19 km/h to 26 km/h.

One appropriate solution to overcome this problem would be the introduction of a Public Transportation Priority System (PTPS), which have not yet been implemented because the headway time does not fit with the signal cycle on the road in the rush hours. Instead, line control signal system considering the bus operating speed has been installed. However, although the PTPS is ineffective in the rush hours, it would have absolutely increased the speed in the off-peak hours.

3.2 Number of passengers

Key Route Bus system characterized by higher operation speed and better punctuality offers convenience over an ordinary bus and is expected to attract more passengers. The survey results confirm this likely result and also notify a shift from the car users by 3% which a notable shift is considering the bus and the private car.

Fig-6 shows the changes in passenger demand for the Key Route Bus No.2 taking the demand in 1988 as 100. The number of passengers for the Key Route Bus decreased to 85.6% in 2003 but this was a smaller decline compared to that of total bus passenger demand of decreasing to 69.8% and Key Route Bus No.1 also decreasing to 79.4% in the same term. So that it can be concluded that the median bus lane has any effect to prevent the decrease rate in bus passengers. Therefore we reach the important conclusion that better service level of the exclusive median lane bus as discussed above was able to maintain the demand on the system better than the other two types of ordinary and exclusive curbside lane bus services in Nagoya.

<Fig-6>

Using the data available to the authors over the 46 sample bus lines, a simple demand model of bus passengers was estimated by the below equation and .the model estimation results are represented in table-4.

Y=b0+b1Xi+b2Xii+b3Z+b4ln(Si)				(1)

Where:

Y: Number of passengers [person / day]

Xi: Population in the catchment area [person]

Xii: Number of students in the catchment area [person]

Z: Average distance of bus service [km/service]

Si: Number of annual bus services per population in the catchment area [services/person]

The catchment area was defined by a circular area within a 500 m radius from the bus stop. The given variables proved statistically significant relations to explain the demand on the buses but better for the ordinary buses than the Key Route Bus Routes. Fig-7 shows the observed and estimated demand for the two Key Routes Lines and three selected ordinary bus lines (A-Line, B-Line and C-Line). The most likely reasons for the underestimation of the Key Route Lines No. 1 and No. 2 are because the number of the ordinary bus lines was more in the total sample and obviously the Key Route Bus system has higher capacity and potential to carry more passengers.

<Table-4>

<Fig-7>

3.3 Change in bus stop access/egress and total trip time

Because the bus stops of median lane bus lines are designed in the central part of the road with longer interval, this may have an adverse effect of increasing the access/egress time of the bus stops and increase the door to door total trip time although higher speed on the exclusive lanes contribute to the shorter trip time. To investigate such an impact, we defined a belt-shaped area with 500m width and 10km. long as depicted in Fig-8. We considered three alternatives of an ordinary bus, a median bus lane Key Route Bus and a subway and calculated the access / egress time by the given equation 2 from each 50m x 50m mesh to point A at the city center. The bus stop intervals and the operation speed and standard deviation of delay time are the actual values derived from the Nagoya City Office Observations (Table-5) and the headways of the each transport mode was accepted to be identical to avoid any biased results arising from the operation schedules.

<Fig-8>

tij=lai60Va+tbj+tcj2+60ldi1000Vdj+10tej					(2)

where;

tij: Access time from mesh  i  to point A [min]

tbj: Waiting time at signals on the way to access bus stops (Key Route Bus) or platform entrance (Subway) [min]

tcj : Frequency of each transport mode j [min]

tej : Standard deviation of delay time per kilometer [s/km]

lai : Distance from mesh i to bus stop or platform station [m]

ldi : Distance from bus stop or station to point A [m]

Va : Walking speed [m/s]

Vd :Operation speed of each transport mode j [m]j

<Table-5>

The results imply that longer intervals between the bus stops increased the access /egress time, but was not large enough to overwhelm the improved running time of the Key Route Bus and therefore the total trip time was found to be fairly shorter than an ordinary bus on the network.

Comparing with any subway improvement, median lane proved its cost-effectiveness as the construction cost is only one hundredth of the subway but the total trip time is improved by 42 %.

3.4 Changes in the road capacity

Traffic capacity on the road network evidently decreases after assignment of one lane as the exclusive bus lane. Particularly the median exclusive bus lane exerts negative effect on the traffic making right turns at the signalized junctions and is the main reason for the difficulty of implementing a median bus lane or Light Rail Transit in Japan. To investigate the reduction of road capacity after the installment of the median lane, we examined three cases; 1) only passenger car traffic, 2) combined passenger car and ordinary bus traffic and 3) car traffic with exclusive median bus lane Key Route (assigned from the ordinary existing lanes on the road) by calculating the number of people as a simple performance indicator.

In the first case with the typical signalization pattern, we simply multiply the total traffic capacity by the average vehicle occupancy (1.3 people/car) and the average peak hour trip speed. For the cases 2 and 3 the average speed of the buses in the peak hour were taken as 12.2 km/hr and 17.9 km/hr, respectively.

Fig-9 shows the capacity changes, separately for two and three lanes. The total capacity was found to be the highest for the combined passenger car and ordinary bus case but the share of the bus was the highest for the third case with an exclusive median bus lane obviously as the consequence of the improved service level by the Key Route Bus System.

<Fig-9>

3.5 Reduction of the environmental load

Since the fuel consumption and environmental load from the buses  can be improved by increased average operation speed, we estimated such effect of the median bus lane by employing the fuel consumption model suggested by Oshiro et al. (2001). Implementation of curbside bus lane reduces energy consumption by 8% and median bus lane reduces it by 13% comparing with ordinary bus (see Table-6).Additionally,an actual 3 % shift from car trips to buses, mentioned earlier, also contributed to the reduction of environmental load by the amount of  370 t CO2/year. Thefore the median bus lane totally reduces the carbon dioxide emission by 607t/year.

<Table-6>

3.6 Effect of housing location changes

Well known interaction between the transport and the land use is likely to encourage housing and commercial location pattern along the Key Route bus line with potential to attract more facilities within the concept of Transit Oriented Development. .

Table-7 presents the population changes since the 1980 separately for the areas within 500m far and the area 500 to 1000m far from each bus stop, and the other accessible along the line.  It should be note that the area has already been developed before introducing median bus lane, and recently facing population decline common and new phenomena in Japan. However the Key Route Bus of Nagoya hardly put any evidence of transit oriented development along its line or nearby the bus stations. The population decline within 500 m zones was less than the zones 500m to 1000 m. but higher than the other zones.

<Table-7>

4. Conclusion

Bus Rapid Transit Systems are regarded as good options of public transport as they provide better services with higher speed and meet higher capacity compared with the traditional buses. BRT systems demonstrate evidently low capital and operation costs per km compared to the railways. They also generate less environmental load because of likely shift from car users and improved speed on the bus network. Therefore BRT has also been acknowledged as one of the best solutions in the developing countries with growing demand and limited financial resources especially in the cases requiring immediate transport network improvements.

Implementations of Bus Rapid Transit Systems have been increasing with a variety of applications of running ways, vehicles, stations and operation systems. However, BRT implementations has been quite rare in Japan and one typical example is the Nagoya Bus Rapid Transit so called here Key Route Bus, our focus in this study, has been operating more than twenty years. The central concern of this paper, to conduct an ex-post evaluation for assessing its performance by a number of indicators has revealed six important results that can be summarized as below:

1) Operation speed and punctuality is much improved. The main contribution is the reduction in the number of passenger cars intruding in to the exclusive bus lane. However, long waiting time and stops at the signals especially in the rush hours still remains to be problematic.

2) The passenger demand was found to be increased by 74% compared with the total number of ordinary bus passengers unless the Key Route Bus system would have been implemented.

3) Although intervals among the bus stops were expanded after the installation of by median bus lane, the overall trip time was reduced because of the improved headway and speed by the Nagoya BRT system.

4) The total road capacity in terms of the total number of passengers declined only slightly although one lane was reallocated as an exclusive bus lane because of the increased number of passengers carried by buses.

5) Research about the impacts of the BRT systems on environment has been rather rare. Here, by employing the bus demand and bus emission models, Nagoya Key Route Bus was found to contribute to environmental load reduction by producing less Carbon dioxide emissions by the amount of 607 t/ year.

6) Unfortunately, Nagoya BRT could not demonstrate any positive impact of Transit Oriented Development that the change of population and other urban facilities was not favoring the idea of BRT supportive land use development.

References

- Nagoya City Office (1986), Report of measurement about Key Route Bus operation effect (in Japanese)

- Hisao SUGINO (1998), Improvement of Bus Transport System in NAGOYA -KEY ROUTE BUS and GUIDE-WAY BUS, Infrastructure Planning Review(Japan Society of Civil Engineers), No.15, pp.639-646 (in Japanese)

- Herbert S. Levinson, Samuel Zimmerman, Jennifer Clinger, C. Scott Rutherford (2002), Bus Rapid Transit: An Overview, Journal of Public Transportation, Vol.5, No.2

- Transportation Research Board (2003), Bus Rapid Transit, Volume 1: Case Studies in Bus Rapid Transit, TCPR Report 90

- Tsutomu YABE, Fumihiko NAKAMURA, Toshiyuki OKAMURA (2005), A Study on Systematization of Performance Measure for Evaluation of Bus Rapid Transit, Proceedings of Infrastructure Planning (Japan Society of Civil Engineers) No.32, CD-ROM (in Japanese)

- Hideki NAKAMURA, Kazufumi SUZUKI (2004), INSPECTOR - The Integrated Network Simulator for Performance Evaluation on Comprehensive Traffic Operation, 11th World Congress on Intelligent Transportation Systems, CD-ROM

- Research Conference of Transportation Problems in the city of Nagoya (1997), The concept and management reconstruction plan of the city of Nagoya transportation services (The 4th report) (in Japanese)

- Nodoka Oshiro, Masayuki MATSUSHITA, Ryoji NAMIKAWA, Hirofumi Onishi (2001), , Fuel consumption rate and a carbon dioxide discharge coefficient  in a vehicle travel (in Japanese), Civil Engineering Journal, Vol.43，No.11，pp.50-55

Fig-1 Key Route Bus System

Fig-2 Key Route Bus Plan in the City of Nagoya

Fig-3 Route of Key Route Bus No.2

Fig-4 Effects of introduction of median bus lane

Fig-5 The speed-improvement and punctuality effects

Fig-6 Changes in passenger demand since Key Route Bus No.2 has been operated

Fig-7 Relationship between the estimated value and observed value in 2004

Fig-8 Virtual belt-shaped area to analyze access/egress time

Fig-9 Results for the capacity of person kilometer change calculations

Table-1 Examples of BRT System (Median bus lane)

|  | city | population (million) | opening year | Separated/ Non-Separated |
| --- | --- | --- | --- | --- |
| Brazil | Curitiba | 1.6 | 1973 | Separated |
| Columbia | Bogota | 6.8 | 2000 | Separated |
| China | Beijing | 13 | 2005 | Separated |
| China | Kunming | 3.7 | 1999 | Non-Separated |
| China | Hang Zhou | 6.4 | 2006 | Non-Separated |
| Ecuador | Quito | 1.6 | 1995 | Separated |
| Indonesia | Jakarta | 11 | 2004 | Separated |
| Japan | Nagoya | 2.2 | 1985 | Non-Separated |
| Korea | Seoul | 9.9 | 2004 | Non-Separated |
| Mexico | Mexico City | 18 | 2005 | Separated |
| Taiwan | Taipei | 2.6 | 2001 | Non-Separated |

Table-2 Service Level characteristics of Key Route Bus (No.2)

| Routes | Operation Speed | Alighting Time | Peak Volume of Buses |
| --- | --- | --- | --- |
| 3 | 20km/h | 30min | 33 /h |

Routes

- Sakae – Hikiyama

- Nagoya Central Station – Hikarigaoka / Idaka Shako

(Above two Routes are operated by Transportation Bureau of City of Nagoya)

- Nagoya Central Station – Shiken-ya etc.(East Area from Hikiyama)

(Above one Route is operated by Meitetsu Bus Co,.Ltd)

Table-3 Delays from the scheduled time

| (per kilometer) | Ordinary route bus (A) | Curbside bus lane (B) | Median bus lane (C) | Improve ment [%] 1-{C/A} | Improve ment [%] 1-{C/B} |
| --- | --- | --- | --- | --- | --- |
| Average delay time per km(sec/km) | 35.8 | 52.1 | 16.3 | 54.4% | 68.7% |
| Standard deviation of Delay Time per km [sec/km] | 22.6 | 26.4 | 16.8 | 25.7% | 36.4% |

Table-4 Parameter Estimation Results

|  |  | Coefficient | t-value |
| --- | --- | --- | --- |
| Population in the catchment area | b1 | 0.00812 | 7.3 |
| Number of students in the catchment area | b2 | 0.439 | 2.56 |
| Average distance of bus service | b3 | 244 | 2.34 |
| Log (Number of annual bus services per population in the catchment area ) | b4 | 4814 | 11.7 |
| Constant term | b0 | -1754 | -1.79 |
| Number of samples |  | 46 |  |
| R2 |  | 0.872 |  |

Table-5 Access/egress time delay analysis

| Mode | Assumed value |  |  |  | Calculated value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Interval of bus stops / stations d [m] | Waiting time at signals tb[min] | Operation speed Vd[km/h] | Standart deviation of delay time te[s/km] | Average access time to bus stops or stations te[min] | Average total trip to point A t[min] |
| 1)Ordinary bus | 300 | 0.00 or 0.88 | 13 | 23 | 4.50 | 33.2 |
| 2)Median bus lane | 600 | 0.88 | 19 | 17 | 5.89 | 26.9 |
| 3)Subway | 1200 | 1.50 | 33 | 0 | 8.31 | 20.2 |

Table-6 Reduction of the environmental load effect

| Estimated Mode Value | Operation speed [km/h] | Fuel consumption [km/L] | CO2 emission [ton-CO2/year] | NO2 emission [ton-NO2/year] |
| --- | --- | --- | --- | --- |
| Median bus lane | 19 | 3.58 | 1,520 | 11.2 |
| Curbside bus lane | 16 | 3.35 | 1,620 | 12.0 |
| Ordinary bus | 13 | 3.10 | 1,750 | 13.0 |

Table-7 Population, changes along the Key Route Bus

| Population | 1980 | 1990 |  | 2000 |
| --- | --- | --- | --- | --- |
| (1) Within 500m zone | 81,930 | 84,073 |  | 81,108 |
| (2) 500-1000m zone | 111,936 | 109,774 |  | 104,064 |
| (3) Others | 169,084 | 186,172 |  | 184,259 |
| Change rate of population | 1980-1990 |  | 1990-2000 |  |
| (1)500m zone | +2,62 |  | -3.53 |  |
| (2)500-1000m zone | -1.93 |  | -5.20 |  |
| (3Others | +10.1 |  | -1.03 |  |