MICROSCOPIC TRAFFIC SIMULATION BASED ANALYSIS FOR CONTRAFLOW OPERATIONS

Qiang Meng and Hooi Ling Khoo

Department of Civil Engineering, National University of Singapore, Singapore 117576

Abstract

This paper addresses an optimal decision problem arising in contraflow operations, which aims to minimize the total travel time in a study area by simultaneously determining the lane reversing configuration and scheduling. After presenting a bi-level programming model for the problem, this paper proceeds to develop a genetic algorithm embedded with a traffic simulation model for solving the proposed bi-level programming model. Finally, an illustrative case study is carried out to test the methodology proposed. Numerical results show that simultaneous determination of contraflow lane configuration and scheduling outperforms solving the two problems - contraflow lane configuration and scheduling – individually.

1. Introduction

Contraflow operation deals with temporarily reversing some lanes of the opposite underutilized roadway to cater for the abrupt traffic flow increment of the busy sides. It is a highly cost-effective dynamic traffic management strategy since significant capacity gains can be obtained without the need to construct additional lanes. Contraflow has long being adopted as an efficient tool to manage the traffic during hurricane evacuation (Theoudoulou and Wolshon, 2004). Due to its benefits, contraflow has been suggested to apply on the urban roadways to alleviate the recurring congestion due to unbalanced demand on both sides of the roadways. In fact, it has been successfully implemented in a few cities such as Kuala Lumpur, Beijing and Vancouver to manage daily traffic condition in their central business district.

To apply the contraflow operation, two decision problems should be examined during the design and planning stage, namely the optimal contraflow lane configuration problem (abbreviated as OCLCP) and the optimal contraflow scheduling problem (abbreviated OCSP). The former (OCLCP) deals with determining which lanes of the candidate road segments that could have their directions reversed in order to minimize a traffic threshold (Meng et. al, 2005) while the latter (OCSP) decides the start time and the duration of the operation (Meng and Khoo, 2006).

With regards to OCLCP, Drezner and Wesolowsky (1997) made an initial attempt to determine the optimum configuration of one-way and two-way routes in the case of flow-independent link travel time patterns. Tuydes and Ziliaskopoulos (2004) formulated the OCLCP for solving traffic congestion as a continuous network design problem. In their standard continuous optimization formulation, capacities of the reversible lanes are treated as continuous decision variables and a system optimum dynamic traffic assignment model without consideration of the selfish behavior of drivers in route choice is involved. Theodoulou and Wolshon (2004) presented an interesting case study regarding freeway contraflow evaluation on the westbound I-10 out of the City of New Orleans. They used the microscopic traffic simulation program, CORSIM, to evaluate two alternative scenarios of the contraflow lane configurations for one road segment along I-10. Meng et al (2005) developed a bi-level optimization model for the OCLCP and solved the model using the hybrid GA-PARAMICS methodology. Lim and Wolshon (2005) studied a variation of OCLCP, which examines the configuration of the contraflow termination point.

With regards to OCSP, Zhou et al (1993) tried to compute an online optimal schedule for the George Massey Tunnel in Vancouver by adopting a fast optimization algorithm. In the study, they developed an intelligent controller comprised a Fuzzy logic pattern recognizer to estimate the flow demand pattern from the partially available flow demand data obtained online from the detectors mounted on the tunnel. They then adjust the online optimal schedule according to the estimated pattern. Xue and Dong (2002) made efforts in improving the estimation accuracy by incorporating least square curve-fitting to remove the random traffic noise. Meng and Khoo (2006) formulated the OCSP as a bi-level programming model and solved the formulation by adopting a hybrid GA-PARAMICS methodology. The model developed by them does not require online data and is suitable to be adopted during the design and planning stage of contraflow operation.

From the existing literature, it is observed that the OCLCP and OCSP are solved separately. The results obtained from either one of them are not a real optimum to the problem. This is because when either one is considered, the other one is always assumed known and fixed. For example, for OCLCP, the schedule of the operation needs to be assumed and fixed. In order to obtain an overall optimum solution for the contraflow operation, both problems need to be considered simultaneously. This means that by solving an optimization model, the lane reversing configuration and the schedule for contraflow operation can be obtained simultaneously. Such problem is termed as simultaneous determination of contraflow lane configuration and scheduling problem (abbreviated as SIMCONS). In SIMCONS, the lane reversing scheme, the start time and the duration of the contraflow operation are the decision variables that need to be determined. The nature of SIMCONS characterizes a typical leader-follower of the game theory (Migdalas, 1995). The leader is the decision maker who decides the configuration, the start time and the duration of the operation while drivers are the followers who are individual decision maker in response to the change of the transportation network.

It is well known that bi-level programming model (Shimizu et al, 1997) can be adopted to formulate the leader-follower phenomenon. Therefore, SIMCONS is formulated as a bi-level programming model. It consists of the upper and lower level sub-problems. The upper level sub-problem is formulated as an integer programming that aims to determine a feasible contraflow configuration and schedule, represented by a vector of binary (0-1) decision variables, such that the total travel time in a study area is minimized. The lower level sub-problem is able to capture the route choice of drivers in response to a configuration and schedule. This paper employs microscopic simulation model as the lower level sub-problem, which is able to capture the dynamic behavior of drivers in great detail, including interactions among vehicles, lane changing and routing choice. In fact, analytical dynamic assignment model or mesoscopic traffic simulation model such as DynaMIT (Ben-Akiva, 2000) can be adopted as the lower level. However, this study chooses microscopic simulation model because SIMCONS is formulated as a lane-based model in which the lane choice logic is also an important feature that needs to be captured. Having employed a third party microscopic simulation model, the objective function of the derived upper level sub-problem does not possess an analytical expression. Therefore, the conventional branch-and-bound algorithm for integer programming problems is no longer available for the proposed model. Given any feasible lane reversing scheme and schedule, the objective function value of the upper problem can be evaluated by implementing the microscopic traffic simulation model. Hence, the famous meta-heuristic method, genetic algorithm (Holland, 1975), is employed for solving the bi-level programming model because these solution methods only require the value of the objective function at each iteration. GA is chosen in this paper in view of the successful applications of GA with microscopic traffic simulation models (Ma et al, 2004, Cheu et al, 2004) in the previous study.

This paper is organized as follows. We will first introduce the bi-level programming model for the SIMCONS. Then we propose a hybrid GA-PARAMICS methodology to solve the model. Next, we will show the robustness of our methodology by adopting an illustrative case study of arterial streets of Singapore. Note that our model is not restricted to arterial streets only but also applicable on the freeways. Finally, comparison to the result obtained by solving the problem separately, i.e. OCLCP and OCSP is carried out to show that SIMCONS is able to produce a lower optimal result compared to the previous two.

2. Bi-level Programming Model

Let  denote a connected and directed transportation network, in which  , the set of nodes, and , the set of links. The sets of origins and destinations are denoted by  and , respectively, and . For an O-D pair ,  and , there is a time-dependent traffic demand denoted by  where  represents a time instant. Each directed link  composes several lanes which are numbered from the shoulder lane to the median along the direction from nodes i to j by integer numbers , where  is the total number of lanes in the link. Figure 1 illustrates an example of the above lane numbering scenario for two links connecting nodes  and.

(Figure 1)

Note that Figure 1 shows the right-hand-drive (kept-left) convention which is used in Japan and most of the Commonwealth countries/territories.  The general formulation provided in this study can also apply to the European and North American driving convention (left-hand-drive, or keep-right) without modification, except the coding of simulation model.

Let also be the set of links in the study area such as Central Business District (CBD), i.e., ,  be the set of candidate links for implementing the contraflow operation. Candidate links in set  are usually identified by the transportation agency or traffic engineers who have information about the possible bottleneck links that may result in the traffic congestion in the study area based on historical traffic flow data. Note that set  may not be identical to set  in practice. Given also a time period denoted by  when the contraflow operation is considered. This time period is discretized into homogeneous interval when each  represents a -minute interval, namely, .  There are three decision variables in the problem and are defined as follows:

: start time (in time interval) of the contraflow operation for link

: end time (in time interval) of the contraflow operation for link

For the sake of presentation, let represent the row vectors of the start and the end time and the binary decision variable for lane reversing scheme for all the links, respectively, namely,,  and . In consideration for the traffic safety and operational feasibility, there are some constraints need to be defined for the problem. Firstly, there must be a minimum,   and maximum,  duration for the contraflow operation. This is expressed by:

where  Note that . The value of   and  can be determined by the authority or engineers according to the traffic condition of the study area. Secondly, the sequence of reversal in travel direction in a candidate link must be consecutively from the median lane to the shoulder lane. This constraint is mathematically described by

In the situation that both links connected by a same node pair are candidate links, , only one of them will be allowed to reverse its travel direction. Such a constraint is expressed as:

where  is any integer number between zero and .

Any  satisfying constraints - is referred as feasible SIMCONS solution. Given a feasible SIMCONS solution, the network topological structure has changed. The total travel time experienced by the drivers in the study area is expressed as follows:

where  is the total number of vehicles passing through the study area during time ;  is the time of the  vehicle leaving the area and   is the time of the  vehicle entering the area. The difference of these timing is the travel time of  vehicle traveling in the study area. This includes any waiting time within a queue if there is any.

To summarize, the bi-level programming model for SIMCONS:

subject to

where  can be evaluated by implementing a microscopic traffic simulation model shown in Figure 2 for any feasible .

(Figure 2)

In terms of terminology in the subject of bi-level programming, the binary integer programming problem expressed by eqns. - is referred to as the upper level problem, and the microscopic model depicted in Figure 2 is called the lower level problem. The objective of the upper level problem is to minimize the total travel time of the study area, eqn.  subject to three kinds of constraints discussed earlier.

3. Solution Algorithm

GAs is inspired by Darwin’s theory of evolution (Holland, 1975). It starts with a population of individuals represented by chromosomes. Individuals from one population are taken and used to form a new generation of population according to their fitness – the more suitable they are the more chance they have to reproduce. Encoding of chromosome and choice of fitness function explicitly depend on characteristics of a problem. For the preceding bi-level programming model, a chromosome is encoded by the row vector of all the binary decision variables, namely,, where  and  and . Note that each gene of the chromosome,   only takes value 0 or 1. The total operation period (minute) is discretised into -minute time interval. The length of the chromosome,  is expressed as eqn.  where  is the total number of candidate links:

The objective function shown in eqn.  is adopted as the fitness function in designing our own GA.

A conventional GA consists of three main steps: selection, crossover and mutation. The selection step attempts to select the best solutions from the population to be parents to generate the possible feasible solutions for next iteration using crossover. The solution with better fitness (objective) function values, tend to be chosen in which they have high probability to be selected to the next generation. The selection method applied in this study is rank selection (Mitsuo and Cheng, 1996). The crossover step is the process of combining the genes of a selected chromosome with those of another to create offspring that inherit traits of both parents. As the binary encoding scheme of a chromosome is chosen in this study, two point crossover (Kobayashi et al., 1996) is used. After crossover and mutation, the generated solution may violate constraints - of the preceding bilevel programming model. Accordingly, the solution needs to be repaired before it is evaluated at the lower level. The following section demonstrates a chromosome repairing procedure.

3.1 Chromosome-Repairing Procedure

Step 1. Perform the following operations for the entire chromosome,  of

links .

Step 1.1. Check each gene of the chromosome, a sub-string of

, from left hand side to right hand side to identify

those first gene that shown value 1 to represent the contraflow

start time for all the candidate links,   .

Step 1.2.  Check each gene of the sub-string , from right

hand side to left hand side to identify those first gene that

shown value 1 to represent the contraflow end time for all

candidate links,  .

Step 1.3. If   or  , generate two random

numbers that fulfill constraint  and assign the smaller

number to , the larger number to .

Step 1.4.  Check each of the gene starts from to . If there is any

gene with value 0, go to step 2. Otherwise, go to Step 3.

Step 2. Perform the following operation for the sub-string  from Step 1. 		Step 2.1.  Assign a value 1 for all the genes after the start time until the

end of the time interval period and assign value 0 for all the

genes before the start time.

Step 2.2.  Assign a value 0 for all the genes after the end time until the

end of the time interval period.

Step 3: Check each gene of the sub-string for chromosome  from Step 2 or Step 1, starting from its left hand side to identify those genes that represent all the lanes of a candidate link, say , which forms a sub-substring, , of the chromosome. For the sub-substring, , let  be the number of genes with the value of one, namely,

The following operation is implemented for the chromosome, with k  numbered from the shoulder lane to the median lane.

Step 4: Let   be the sub-string of chromosome generated from Step 3. For each link, perform the manipulation on this chromosome as follows.

Case 1: the substring, , associated with link  precedes the substring, , related to link  in the sub-string, .  If there is at least one gene with value of 1 in the sub-string, the value of all genes in another sub-string  is set to zero, i.e.,

Case 2: the sub-string, , associated with link  follows the sub-string, , related to link  in the string, .  If there is at least one gene with value of 1 in the sub-string , the value of all genes in substring  is set to zero, i.e.,

The choice of Case 1 or Case 2 is done randomly. Let  be an indicator to determine the choice of the case, where   ; if , Case 1 is chosen while Case 2 is chosen if . It is easy to verify that the chromosome repairing procedure yields a feasible contraflow lane configuration solution, i.e, a solution fulfills constraints - of the bilevel programming model.

3.2 Genetic Algorithm

Step   0. (Initialization) Randomly generate a population of   M  chromosomes.

Step 1. (Chromosome repairing) For each infeasible chromosome in the population generated in Step 0, apply the chromosome repairing procedure. The infeasible chromosome in the population is then replaced by a feasible one

Step 2. (Calculation of the fitness function) For each chromosome  in the population, the value of fitness function defined by eqn.  is evaluated by implementing the microscopic traffic simulation model shown in Figure 2.

Step 3. (Generation of a new population). Repeat the following four sub-steps until the new population is completed.

Step 3.1. (Selection) According to the fitness functions value evaluated in Step 2, use the rank selection method to choose two parent chromosomes from the population.

Step 3.2. (Crossover) With a crossover probability, denoted by , cross over the parents to form a new offspring according to the one point crossover method . If no crossover is performed, offspring is the exact copy of the parents

Step 3.3. (Mutation) With a mutation probability, denoted by , mutate new offspring at selected position in chromosome.

Step 3.4. (Repairing) If the offspring results in an infeasible solution of the upper level problem, repair it by executing the chromosome repairing procedure.

Step 4. (Stopping criterion). If a stopping criterion is fulfilled, then terminate, and output the best solution from the population. Otherwise, go to Step 2.

The termination criterion used in this study is to terminate GA when it achieves a maximum number of generations specified. Note that the performance of GA may rely on the size of population and crossover and mutation probabilities.

4. An Illustrative Case Study in Singapore

In this section, an example case of Singapore CBD area is adopted to test the methodology proposed. It adopted PARAMICS (Quadstone, 2005) as the microscopic traffic simulation model of the lower level problem in the bilevel programming model for the OCSP.

4.1 Network Coding

The network used in the case study covered the CBD area of Singapore. It was coded in PARAMICS  and was represented by 2046 links, 1050 nodes, and 36 zones, as shown in Figure 3 has been calibrated by Lee et al. (2004). Among these links, 18 links numbered from 1 to 18 that indicated by hatched links in Figure 3 formed our study area, i.e.,  within which the GA’s fitness function at the upper level problem was calculated. Among these links, only 9 links have been recognized as the candidate contraflow links, i.e. . In the real situation, these links can be chosen based on the traffic condition, for instance, those congested links. So, the length of the chromosome with one hour simulation period sliced into one-minute interval is . For each candidate link, there are three candidate lanes. So, the total length of the chromosome is 567 gens. The OD demand of the network is assumed values. In order to emulate the dynamic departure time choice, the demand is loaded onto the network at 5-minute interval according to a profile which resembles normal distribution.

(Figure 3)

In this case study, the dynamic feedback traffic assignment method in PARAMICS was chosen to reflect the dynamic behaviors of drivers, including the route choice, in response to the change of transportation network topological structure. Under this method, PARAMICS updates the link costs that include the turning cost. The turning delays are calculated at defined feedback period, i.e. at every one minute interval. PARAMICS classifies the drivers in its model into two groups namely familiar and unfamiliar drivers. It assumes that only familiar drivers are incorporated with full knowledge of the entire network and are able to find alternative routes to avoid congestion. To do this, the route cost table of the familiar drivers is updated at every time interval specified so that they can react to the congestion. In this study, the dynamic feedback period is set as 1-minute.

4.2 Contraflow Operation Modeling

PARAMICS does not support the direct emulation of contraflow operation. The traffic on the links is not allowed to be reversed on the network. To imitate the contraflow operation shadow lanes are created in the opposite direction of the candidate links. Due to this reason, prior to the simulation, we need to identify the candidate links. A sample link of  is taken to illustrate the shadow lanes concept in coding of contraflow lane in PARAMICS. The candidate lanes for contraflow are lane 1 and 2 of link. Two shadow lanes in the opposite direction, i.e. lane A and lane B of link  were created to represent the candidate lanes. Figure 4 schematically depicts this sample link. Lane A is the shadow lane of lane 2 while lane B is the shadow lane of lane 1. If the travel direction of lane 2 of link  is to be reversed, then lane 2 will be closed to traffic and lane A of link will be opened. The same also goes to lane 1. Thus, lane 2 and lane A work as a pair to imitate the contraflow operation in which both of them are not allowed to be opened simultaneously. Certainly, in the real network layout, the shadow lanes do not exist.

(Figure 4)

As the problem deals with lane-based model, the lane choice rule and setting in PARAMICS is crucial especially when we deal with shadow lanes. In PARAMICS the lane choice logic is modeled using the signposting/hazard model. The signposting defines any hazard condition in the network and vehicles will react according to this information. Hazards in the network can be any of the following conditions: right turn or left turn at the intersection, incidents, and lane drop or lane closure. Vehicles will be able to observe the hazard when they are approaching to these conditions and able to do the correct decision based on other models such as lane changing model and gap acceptance model. This understanding is crucial as we should know how the vehicles will deal with shadow lanes. In other words, when the shadow lanes are opened for the vehicles, they will have equally opportunity to be used by the vehicles (equally spread characteristic in the lane changing model). Extra care is taken for these lanes to deal with the lane turning choice at the intersection to avoid vehicles cross each other and behave odd in the network. Since the driver behavior during contraflow operation is vague in literature i.e. whether the drivers will more cautious and slow down their speed is not known in reality. As a consequence this behavior is neglected in the model although it can be done by for example specifying a percentage reduction in free flow speed when the contraflow is in operation.

4.3 Application Programming Interface (API) Programming

PARAMICS provides application programmer interface (API) which enable users to enhance its functions and communicate with the core models. To use the API, users need to write their source code in C or C++ and use the provided API library to access to the core models. Basically, PARAMICS provides four classes of interface functions, namely, getting functions, override functions, setting functions and extending functions. The output from PARAMICS is input into the GA program to calculate the fitness value while the output of the GA (i.e., chromosomes) is captured by the PARAMICS to amend the network’s contraflow scheduling and carry out the route choice behavior simulation.

- 4.4 Simulation Replications Using Statistics Analysis

PARAMICS is a stochastic simulation model. The simulation process is governed by random numbers. For example, it uses random numbers to simulate vehicle releases, aggressiveness, awareness, route choice and others. Hence, the result of one simulation run may not be representative of the typical outcome. The results may change substantially with different random number seeds. Multiple runs are needed to produce reliable results. The number of simulation replications is determined using statistical analysis. This is because collecting simulation results is similar to collecting samples from a population. We need to specify , the confidence level required and , the desired relative error between sample mean  and the true population mean to determine the sufficient sample size. The formula used to compute this is:

where is sample standard deviation;  is the critical value of the t-distribution at significance level . The t-distribution is adopted in eqn. because the population variance is unknown and the initial number of replication is small. In this case, we can set the initial replications as 5. Then,  and are calculated. Using =0.05 and , the required number of replication  is calculated using equation . If the sample size is not adequate, one additional simulation run will be performed and the whole process is repeated until the sample size is sufficient.

4.5 Measure of Effectiveness

The total travel time,  of the study area for run number  is expressed by:

where is the simulation time when vehicle exit link and is the simulation time when vehicle enter link . The difference between the in and out time is the travel time spent by the vehicles in the links. The travel time calculated including the turning delay and queuing delay encountered by the vehicles. The delay results from capacity loss during the operation can be captured since eq.  calculates the travel time for vehicles traveling on both candidate links and their opposite lanes. The fitness function of GA at the upper level is calculated as an average of total number of replications done for each chromosome according to eqn.. This is to ensure that the result obtained is statistically reliable. The average system total travel time is expressed by eqn. . Note that eqn.  is the fitness function of the upper level which is identical with eqn. .

4.6 Results

By setting the GA parameters, i.e. , , and , the optimal solution for the SIMCONS is obtained and is shown in Figure 5. It can be observed that the total travel time at the convergence is 98.6 veh-hr which is about 12.8% reduction compared to the initial solution. It is unfortunate that we are unable to prove that the objective function value and solution obtained is the optimal solution. This is because until to date there is neither theoretical theory (manually enumeration is impossible due to large amount of feasible solutions) nor field result available for comparison. But we are able to show that the final objective function value is lesser than the initial solution. The optimal contraflow configuration and schedule is shown in Table 1 and Table 2 respectively. The GA parameters value mentioned beforehand is the best combination of all the values tested. The GA-simulation program is run repeatedly using various different combination values of the parameters and the best result is presented herein.

(Table 1)

(Table 2)

(Figure 5)

A comparison with the result obtained by considering the problem individually is also carried out. Figure 6 shows the convergent trend and objective function value at optimal for OCLCP, OCSP and SIMCONS respectively. It is clear that the real optimal is achieved when both problems, i.e. OCLCP and OCSP is considered simultaneously. OCSP has the highest optimal objective function value among the three, i.e. 176.7 veh-hr (Meng and Khoo, 2006) followed by OCLCP which objective function is 120 veh-hr (Meng et al, 2005).

(Figure 6)

An issue associated with the methodology proposed is the computational time issue. Since GA requires a significant number of iterations to converge, this yields the long computational time issue. It takes about two minutes to complete a simulation run. However, this issue can be tackled by using distributed and parallel computing technique (Ma et al., 2003). Another issue that woth mentioning is the “paired link” issue. For the sake of practical implementation and the safety consideration, a road which consisted of more than one candidate links should have the same SIMCONS solution. These links are called “paired link”. For an instance, link 2 and 6 in our case study is geographically connected and should have the same contraflow lane configuration and schedule solution. To tackle this issue, only a small alteration needs to be done on our GA coding. We can either code these links as one link or add extra constraints to force these links to have same solution. This will not affect the applicable of the proposed methodology.

5. Conclusions

This paper discusses about solving the contraflow problem, that is the contraflow configuration and scheduling problem simultaneously. The problem, SIMCONS is formulated using the bi-level programming model in which the upper level is an integer programming model that aims to minimize the total travel time of the study area while the lower level is a traffic simulation model that measures the response of drivers to the solution proposed. The model is solved using hybrid GA-PARAMICS methodology. It is shown that solving SIMCONS gives an optimal objective function value that is lower than solving the problems individually, i.e. OCSP and OCLCP. The future work that can be done is to try using other meta-heuristics algorithm such as Tabu Search or Simulated Annealing.

References

Ben-Akiva, M. (2000) DynaMIT: A simulation-based system for traffic prediction and guidance generation, Massachusetts Institute of Technology, United States.

Cheu, R.L., Wang, Y. and Fwa, T.F. (2004) ‘Genetic algorithm-simulation methodology for pavement maintenance scheduling’, Computer-aided Civil and Infrastructure Engineering, 19(6), pp. 446-455.

Drezner, Z. and Wesolowsky, G. (1997) ‘Selecting an optimum configuration of one-way and two-way routes’, Transportation Science, 31(4), pp. 386-394.

Holland, J.H. (1975) Adaptation in natural and artificial systems, University of Michigan Press, Ann Arbor.

Kobayashi, S., Tamaki, H. and Kita, H. (1996) Multi-objective Optimization by genetic algorithms: A review. Proceedings of IEEE International Conference on Evolutionary Computation, pp. 517-522.

Lee, D.-H., Wang, H., Cheu, R.L. and Teo, S.H. (2004) ‘Taxi dispatch system based on current demands and real-time traffic conditions’.  Transportation Research Record: Journal of the Transportation Research Record, No.1882, TRB, National Research Council, Washington, D.C., pp. 193-200.

Lim, E. and  Wolshon, B. (2005) ‘Modeling and Performance Assessment of Contraflow Evacuation’. CD-ROM. Paper presented in the 84th Transportation Research Board Annual Meeting, Washington, D.C.

Ma, W., Cheu, R.L. and Lee, D-H (2003) ‘A two-stage hybrid genetic algorithm-traffic assignment-distributed simulation methodology for lane closure scheduling’. CD-ROM. Paper presented in the 82nd Annual Meeting of the Transportation Research Board, TRB, Washington D.C.

Ma, W., Cheu, R.L. and Lee, D-H. (2004) ‘Scheduling of lane closures using genetic algorithms with traffic assignments and distributed simulations’, Journal of Transportation Engineering, 130, pp. 322-329.

Meng, Q., Khoo, H.L. and Cheu, R.L. (2005) ‘Microscopic traffic simulation models based optimization approach for the contraflow lane configuration problem’. Paper submitted for the consideration for publication in ASCE Journal of Transportation Engineering.

Meng, Q and Khoo, H.L. (2006) ‘Model and algorithm for the optimal contraflow scheduling problem’. Paper accepted for presentation in the 86th Transportation Research Board Annual Meeting, Washington, D.C.

Migdalas, A. (1995) ‘Bilevel programming in traffic planning: models, methods and challenge.’ Journal of Global Optimization, 7, pp. 381-405.

Mitsuo, G. and Cheng, R. (1996) Genetic algorithms and engineering design. John Wiley, New York.

Quadstone (2005). PARAMICS 5.1 Modeler reference guide

Shimizu, K., Ishizuka, Y. and Bard, F. (1997) Nondifferentiable and two-level mathematical programming. Kluwer Academic Publishers, Boston.

Theodoulou, G. and Wolshon, B. (2004) ‘Alternative methods to increase the effectiveness of freeway contraflow evacuation’,  Transportation Research Record, 1865, pp. 48-56.

Tuydes, H. and Ziliaskopoulos, A. (2004) ‘Network Re-design to Optimize Evacuation Contraflow’. CD-ROM. Paper presented in the 83rd Transportation Research Board Annual Meeting, Washington, D.C.

Xue, D. and Dong, D. (2002) ‘An intelligent contraflow control method for real-time optimal traffic scheduling using artificial neural network, fuzzy pattern recognition, and optimization’,  IEEE Transaction on control systems technology, 8(1), pp. 183-191.

Zhou, W.W., Livolsi, P., Miska, E.,  Zhang, H.,  Wu, J. and Yang, D. (1993) ‘An intelligent traffic responsive contraflow lane control system’, IEEE-IEE Vehicle navigation & Information systems Conference, Ottawa, pp.174-181.

List of Tables and Figures

Table 1    Optimal Lane Configuration for SIMCONS

Table 2    Contraflow Schedule for SIMCONS

Figure 1   Lane Numbering Scenario

Figure 2   Lower Level Problem of the Bilevel Programming Model

Figure 3   Study Area and Candidate Links

Figure 4   Shadow Lanes and Lane Logic in PARAMICS

Figure 5   Convergent Trend of GA for SIMCONS

Figure 6   Comparisons for OCLCP, OCSP and SIMCONS

Table 1 Optimal Lane Configuration for SIMCONS

| Link No | Lanes to be reversed | Link No | Number of lanes to be reversed |
| --- | --- | --- | --- |
| 2 | Lane 2, Lane 3 | 12 | Lane 3 |
| 4 | Lane 3 | 14 | Lane 2, Lane 3 |
| 6 | Lane 3 | 16 | Lane 2, Lane 3 |
| 8 | Lane 3 | 18 | Lane 2, Lane 3 |
| 10 | Lane 3 |  |  |

Table 2 Contraflow Schedule for SIMCONS

| Link | Start Time (min) | End Time (min) | Duration (min) |
| --- | --- | --- | --- |
| 2 | 7 | 51 | 44 |
| 4 | 19 | 57 | 38 |
| 6 | 21 | 54 | 33 |
| 8 | 13 | 58 | 45 |
| 10 | 26 | 58 | 32 |
| 12 | 15 | 58 | 43 |
| 14 | 5 | 41 | 36 |
| 16 | 5 | 53 | 48 |
| 18 | 7 | 54 | 47 |

Figure 1 Lane Numbering Scenario

Figure 2 Lower Level Problem of the Bilevel Programming Model

Figure 3 Study Area and Candidate Links

Figure 4 Shadow Lanes and Lane Logic in PARAMICS

Figure 5 Convergent Trend of GA for SIMCONS

Figure 6 Comparisons for OCLCP, OCSP and SIMCONS

 Corresponding author.

E-mail address: cvemq@nus.edu.sg (Q. Meng)