ANALYSIS OF DRIVERS’ BEHAVIOR CONSIDERING COMPLIANCE WITH TRAFFIC INFORMATION AND PROVISION OF FEEDBACK

By

Jose Alberto VELEZ

Graduate School of Science & Engineering

Saitama University

255 Shimo-Okubo, Saitama, 338-8570 Japan

E-mail: jav94@yahoo.com

Kunihiro SAKAMOTO *

Research Associate

Graduate School of Science & Engineering

Saitama University

255 Shimo-Okubo, Saitama, 338-8570 Japan

Fax: +81-48-855-7833

E-mail: sakamoto@dp.civil.saitama-u.ac.jp

Hisashi KUBOTA

Professor

Graduate School of Science & Engineering

Saitama University

255 Shimo-Okubo, Saitama, 338-8570 Japan

Fax: +81-48-855-7833

E-mail: hisashi@dp.civil.saitama-u.ac.jp

* Corresponding author.

Paper submitted to the 11th  WCTR.

Submission date 1 May 2007

Abstract

In this research we carried out a field experiment in which RP data was collected in order to investigate: the influence of pre-trip information on departure time choice, and compliance with en-route information advice, among others. We introduced the concept of feedback and our main objective is to examine the effect of this feedback information on drivers’ choices. Findings show that in this experiment, drivers with feedback had no significantly better outcomes choosing departure time but feedback does have the potential to increase drivers’ compliance rate with information advice.

1. INTRODUCTION

The problems caused by congestion have been addressed by traffic engineers through the use of computing and communications technologies during last decades. Many kinds of systems belonging to what became known as Intelligent Transport Systems (ITS) have been increasingly deployed in many cities in some European countries, as well as US and Japan, particularly during the 1980s and 1990s.

Traveler Information Systems are one of the components of ITS designed with the purpose of collecting, processing, and providing information to travelers (traffic information to drivers and transit information to transit users) that will make them aware of changes in travel conditions, allowing them to make more informed decisions that will ideally lead to a more efficient distribution of modes, departure times, and routes, and therefore to a reduction in traffic congestion, which is the ultimate goal of traffic engineers.

There is a classification according to the two principal stages in which traffic information is provided: (1) Pre-trip travel information and (2) En-route driver information. Among en-route systems, in-vehicle navigation systems are capable of giving drivers turn-by-turn instructions, either visually (detailed maps), or audibly, guiding them from the input origin to the input destination through the route with least mileage or time. Along the way, the system can warn drivers about congested roads and unexpected traffic incidents utilizing VICS technology. VICS stands for Vehicle Information and Communication System, this innovative system initiated operation since April 1995 in Tokyo area (CRDBBRI, 2002) enabling drivers to receive real-time road traffic information about congestion and regulations. This information is edited and processed by a Vehicle Information and Communication System Center, and shown on the navigation screen in different levels using either text or graphical forms (See Figure 1.1). Japan is well ahead of other regions (i.e. U.S. and Europe) regarding development and use of route guidance systems, with service areas covering the whole country and an accumulated number of VICS units in the market that has reached 9.1 million in parallel with 14.5 million car navigation devices by the end of the fiscal year 2003 (Makoto, 2004).

+++insert Figure 1 around here +++

We believe there is a need to continue investigating whether and how drivers respond to traffic information, especially using a methodology that involves a natural setting with actual vehicles. This paper focuses on the context of repetitive trips to the same destination (commuting trips from home to school or work), in which drivers’ behavior may be mainly limited to choosing a departure time for the journey and then, choosing a path between the origin and the destination.

One aspect of the departure time choice which is necessary to investigate, is the commuters’ switching decisions on daily departure time in response to experienced congestion and arrival times to their destinations. Commuters are supposed to behave searching for an acceptable departure time that will make them reach their goals.

As for route switching, it is believed that en-route information systems, such as VICS, help drivers to avoid congestion and choose better routes (routes with less travel time) by diverting from the congested links in the network. However, if en-route information advice (i.e. the suggested route) does not meet the driver’s preferences or the advice is incorrect, drivers are very likely to ignore the information and the system becomes ineffective. Mehndiratta et al. (2000) also says there are many people holding fatalistic attitudes about traffic and are skeptical about the ability of information to improve their travel experience. Reviewing the literature, we concluded there is a need to investigate the factors influencing drivers’ compliance with en-route information.

In this paper the concept of feedback is used. This feedback refers to a table summarizing the trip outcomes (e.g. travel time, arrival time, etc) of a group of drivers, that is given at the end of the journey to the same group of drivers, so they can use it as a reference information to make next day trip’s decisions.

The main objective of this research is to investigate the effect of this feedback information on departure time choice and on the compliance rate with the en-route information advice in a real environment. Regarding departure time choice this feedback is considered a sort of pre-trip departure time reference information. The mentioned en-route information is comprised of VICS information complemented with predictions of travel times.

The paper has been divided into five main sections. In section 2, we extend further the panorama about ATIS by making a review of some studies conducted on issues related with these systems and their influence on driving behavior. Section 3 explains the methodology used in this research followed by section 4 which serves to present the findings of the analyses conducted and brief discussions of the results. Finally, section 6 offers conclusions of our research.

2. REVIEW OF LITERATURE

2.1 ATIS

For ATIS to fulfill its commitment it is imperative that the provided information is first acquired and then used, meaning that an adjustment in travel decisions should be made based on the travel information received. Consequently, it becomes necessary to study the travelers’ decision-making process related to accessing travel information and the influence of the information on traveler choice behavior. In this order of ideas, customer demand for ATIS traffic services can be based on four factors according to Lappin (2000): (1) Characteristics of the traveler, (2) Individual trip characteristics, (3) Regional traffic context, and (4) Characteristics of ATIS services. These same factors are found to influence the traveler response to the information.

Another relevant study is done by Targa et al. (2003) who devised a conceptual model (depicted in Figure 2) with three major stages in making repetitive travel decisions: (1) Information technology access/ownership, (2) Dynamic travel information acquisition, and (3) Change in traveler behavior. The results from the San Francisco area behavioral survey with 1000 respondents, which served as base for this study, show that 96% of respondents had access to at least one information device, 66% of all respondents received travel information and just 33% actually changed their routine travel decisions.

+++insert Figure 2 around here +++

2.1 Driving behavior

Regarding driving behavior, Adler & McNally (1994) described it as the decision-making process characterized by three phases, pre-trip planning, en-route assessment and adjustment, and post-trip evaluation. During post-trip evaluation, drivers update their knowledge pertaining to the travel system based on information gathered from the trip just completed and compare it with prior experiences. This updated perception will influence the pre-trip planning decisions for future trips.

When focusing on commute trips which are repetitive trips to the same destination (either work or school), the decision-making process is reduced to consider only two choices: departure time and route.

2.2.1 Departure time choice

Commuters usually like to have a safety margin to protect themselves against lateness at work; thus, they think in advance of arriving at their offices some time earlier; this is referred as Preferred Arrival Time (PAT) by Mahmassani & Liu (1999), among others, and it is found to be an important determinant of commuter behavior dynamics.

McFadden & Talvitie (1977) analyzed the trip timing decisions for travel to work by automobiles. They used the data from the Urban Travel Demand Forecasting Project (UTDFP) to estimate the parameters of a simple empirical model for the probabilities of drivers rescheduling their trips. The study used the utility maximization principles and multinomial logit model. The variables for the model were arrival time to work, official work start time, on-vehicle time. The model may be viewed as expressing the subject's internal tradeoff of time ‘wasted’ through arriving earlier than necessary, versus the increased travel time in the peak and the probability of arriving late.

Another relevant study was done by Small (1982), who modeled arrival times of car commuters in the San Francisco Bay Area. He studied the scheduling of trips at the individual level. The sample consisted of 527 auto commuters in the Bay Area. A number of socioeconomic and transportation variables, such as family status, occupation, mode of transportation, and work hour flexibility, were used. The results indicated that people were willing to shift their schedules by one or two minutes earlier if they saved some travel time.

2.2.2 Route choice

Jan et al. (1999) reported on the use of GPS data to understand route choice. Their data set was from 216 drivers over one-week period. They concluded that the path chosen on a trip most often differs considerably from the shortest path across the network and also that travelers habitually follow the same path for the same trip. Navigation and en-route information systems are then expected not just to expand drivers’ route knowledge, but to make them aware of unbalances on the roads in order for them to divert route; thus, wasted mileage and time is reduced.

Hato et al. (1995) conducted an SP survey among 852 company employees working in Tokyo to evaluate the influence of past information (expected travel time and intended route before departure based on drivers’ prior experience with the routes involved), present information (congestion encountered in route) and future information (advised travel time and length of congestion) on drivers’ route choice behavior. This hypothetic model is shown for better understanding in Figure 3. Results from this study gave evidence that the intended route had an inertia effect on route choice after the provision of information.

+++insert Figure 3 around here +++

To summarize the factors influencing route diversion under traffic information, we refer Peeta & Yu (2003) and Kim & Chon (2005) and classify these factors in five main categories: (1) Driver Attributes; (2) Route Attributes; (3) Situational Factors; (4) Information; (5) Prior Experience.

An interesting hypothesis was examined by Fujii & Kitamura (2000) who investigated the effects of information acquisition and driving experience using two concepts: information dominance and experience dominance. Experience dominance states that the influence of generic information weakens as the driver gains more driving experience, while information dominance implies that information effects becomes larger as the driver uses more information.

Regarding the subject of compliance with the information, a web-based experiment was conducted to find the factors affecting driver compliance. As a result, the variables: ‘confidence in alternate route’ and ‘travel times savings’ were proven statistically significant. Srinivasan & Mahmassani (2000) proposed that observed route choices in the presence of ATIS are a consequence of two underlying behavioral mechanisms: compliance and inertia. The influence of these mechanisms on route choice behavior was modeled using a driving simulator. Increased compliance propensity was seen with increased trip time savings and lower switching costs. However, the compliance propensity decreases with inaccurate information. Finally, Chen & Jovanis (2003), using a driving simulator as well, found that the driver’s experience with the system on a particular day affects driver compliance more than the experience on a particular location for several times.

3. RESEARCH METHODOLOGY

3.1 Collection of data

Most studies reviewed in the previous section are based on data collected by SP/RP surveys or route choice simulators. As Jackson (1994) did, we believe there is a need for experimentation which studies driver behavior in the presence of traffic information in as natural a setting as possible, over a period of time and, using real drivers. That is the reason why we decided to collect data through a field experiment. We also collected data in other ways to complement the data from the field experiment as described below:

- Before Experiment Questionnaire: Drivers participating in the field experiment filled this questionnaire before the experiment began. The purpose of this questionnaire was to collect data about the drivers’ characteristics (e.g. socio-demographics, regular usage of traffic information, opinion about traffic information, etc) to complement the data collected in both the field experiment and the travel diary survey for the analyses.

- Travel Diary Survey: Each day of the experiment, drivers had to travel in the morning, from their own homes to the gathering place. They were asked to fill a daily questionnaire about their departure time and route switching decisions along with other data such as whether they checked pre-trip and en-route traffic information, congestion condition on the route, etc. This data was used to develop a departure time switching model in order to bring some light about which factors affect drivers’ switching behavior.

The field experiment was carried out on December 2005, from Monday 19th to Thursday 22nd. It basically consisted in simulating the morning commuting trip to Saitama University so that we could observe how drivers responded to traffic information provision when they commuted to their destination.

The same course, which takes between 25 and 45 minutes, depending on traffic conditions, was driven daily for 4 days in a row. The objectives of each day were the same, but drivers could change their choices of either departure time or route according to their experience running the course, or the advice provided by the traffic information.

Drivers for the experiment were recruited through a popular employment information website in Japan. The elevated cost that implies hiring drivers with their cars for the experiment did not allow us to count with such a big number of subjects to analyze, the sample size was rather small. Once we finished the recruiting stage, all participant drivers were divided into two main groups of study, one corresponding to Departure Time Choice and the other corresponding to Route Choice as can be seen in Table 1 and Table 2.

+++insert Table1 and Table 2 around here +++

As shown in Figure 4, the road network on which these drivers ran was controlled by fixing an origin point and also by restricting it to the use of only six alternative routes of approximately equal length. Among the roads comprising the network, there was only one which can be considered a highway due to its width, consisting of three lanes per direction, whereas the rest of the roads mostly have one lane per direction. Four CheckPoints (CP) were established; these represent the intersections where group II drivers were allowed to switch between alternative routes.

+++insert Figure 4 around here +++

3.1.1 Experiment’s Flow for Group I: Departure Time

We began our experiment with the assumption that drivers’ pre-trip departure time decision is based on the previous day experience (Jou, 2001). Based on the reviewed literature, we can say that the probability that a traveler will switch departure time the next time he makes the same journey increases with the difference between actual and preferred arrival times. Preferred arrival time is the time a person wishes to arrive at his destination, for instance someone usually likes to get 5 or 10 minutes before his work start time.

Applying this concept to the case of driving commuters in our experiment, we established two main goals for them: Reaching the destination the closest possible to their preferred arrival time and trying to shorten travel time. These goals were represented by a SCORE defined as follows:

+++insert Equation 1, 2,3 and 4 around here +++

Where i represents each driver and d represents each day of the experiment. Thus, drivers were told to minimize the SCORE as much as possible. The variable MinutesEarlier represents the amount of minutes each driver usually likes to arrive to their office or school before his work or class start time; 8:30 am was set as the work start time for this experiment.

Route assignment remained fixed for all four days of the experiment and drivers were not provided with en-route information about traffic conditions. The only way they had to accomplish the established goals was by switching departure time each day, trying to make an appropriate choice.

Each day, after all drivers arrived at the destination, the data on actual departure and arrival times were collected to calculate the Score for each driver and then a summary table of the day’s outcomes was prepared. This summary table is what we call Feedback Type 1 or Departure Time Reference Information and it was distributed among drivers from sub-group I–A only. This feedback sheet looked like the Table 3.

+++insert Table 3 around here +++

The feedback sheet was organized by route and by ascending order of preferred arrival time in order for drivers to find relevant information for them in an easier way. The idea behind the feedback was that it would help drivers to choose an appropriate departure time for the following day that would allow them to achieve the goal of minimizing their own Score by referring departure times of those drivers who have a similar preferred arrival time and got fewer score on the same routes.

3.1.2 Experiment’s Flow for Group II: Route

All drivers included in this group had almost the same departure time from the origin point (around 7:50 am each morning) and they also had two main goals: Shortening travel time and reaching the destination before 8:30 am. For this group, preferred arrival time is not taken into account because they can not choose departure time. The only way these drivers have to accomplish the established goals is by switching between alternative routes.

After departure and before reaching each of the CheckPoints, drivers could consult two types of traffic information: the one provided by VICS, and travel time predictions for all different routes. These travel times were calculated by a forecasting team located at the University, using the information about congestion provided by VICS and estimating delaying times for each route.

During the four days of the experiment, all drivers from Group II were accompanied by a Co-Driver who was in charge of giving them the forecasted travel times for all alternative routes before reaching each CheckPoint and recording all necessary data.

Each day after all drivers arrived at the destination, the data recorded by co-drivers were collected to prepare a summary table called Feedback Type 2. This table showed all drivers’ averaged travel times and accumulated rate of compliance with information advice sorted by ascending travel times. It was distributed among drivers from sub-group II–A only, before they began to run the course each day.

When the driver followed the direction which included the route that according to the predictions had the shortest travel time, it was accounted as the driver complied with the information advice in this CP. In order to calculate the rate of compliance we used the following equation.

+++insert Equation 5 around here +++

An example of the Feedback Sheet Type 2, which was distributed prior the first day of the experiment, showing basically an idea of expected results, can be seen below in Table 4.

+++insert Table 4 around here +++

The idea behind the feedback was to make drivers know whether those drivers complying with the information advice at higher rates were getting better outcomes (i.e. shorter travel times) and if so, this would strengthen their trust in the system and encourage them to comply more next time they ran the course (see Figure 5).

+++insert Figure 5 around here +++

3.2 Analysis Methods

3.2.1 Logistic Regression

The data obtained from the travel diary questionnaire was used to analyze the decision of switching departure time from one day to the next. The dependent variable in this case is a dichotomous one, hence; we used logistic regression to estimate a binary logit model. Despite we are dealing with a repeated observations issue since we took data from each driver in three occasions (the travel diary recorded 4 days, but we cannot observe the dependant variable in the first day), the possible effect of heterogeneity was ignored as it was done by Bunch et al. (1993) because in a small number of repeated observations from each individual the properties of parameters estimates themselves do not rely on the strict independence assumption and since we do not counted with so many subjects, the benefits of using a much larger pool of data more than overweighed this concern.

The probability of a particular outcome in the logit model theory is represented by the following expression (cumulative logistic distribution function):

+++insert Equation 6 around here +++

Where X represents a regressor for subject i, and β represents its parameter.

If Pi, the probability of the outcome Y=1, is given by (6), then (1 - Pi) is the probability of the outcome Y=0, and Pi /(1 - Pi) is simply the odds ratio in favor of outcome Y=1; the ratio of the probability that outcome Y=1 will happen to the probability that it will not happen:

+++insert Equation 7 and 8 around here +++

Equation (8) is obtained by taking natural log of (7). L, the log of the odds ratio which is not only linear in X, but also linear in the parameters. L is called the logit, and hence the name logit model.

3.2.1 Linear Mixed Models

We used the results form the field experiment to examine the effect of feedback on drivers’ behavior. We considered that a simpler analysis using a two by two table in a randomized experiment (showing the dichotomous variables feedback/compliance and feedback/change in departure time) would not be adequate, since the subjects in the groups are not homogenous and there are other factors affecting their choices besides the existence of feedback, therefore; it was better to develop a model that will take into account the effect of several variables at the same time. The model to choose for the analysis had to deal with the repeated measures issue because the subjects in the field experiment were surveyed over time in repeated occasions.

When we have repeated measures; a change in reliability shows up as different correlations between measurements. The Linear Mixed Models procedure provides the flexibility of modeling not only the means of the data but the covariance matrix as well. It expands the general linear model, so the data are permitted to exhibit correlated and non-constant variability. The term mixed refers either to the fact that we are modeling a mixture of means and covariances, or (same thing) to the fact the model consists of a mixture of random and fixed effects. The fixed effects model the mean of the dependent variable. The random effects model the covariance structure of the dependent variable

The dependent variable should be quantitative whereas independent variables can be both categorical (known as factors) and quantitative (known also as covariates). Subjects and repeated variables may be of any type.

The mixed model generalizes the standard linear model as follows:

+++insert Equation 9 around here +++

In this expression, y represents a vector of observed data, β is an unknown vector of fixed-effects parameters with known design matrix of independent variables x, γ is an unknown vector of random-effects parameters with known design matrix z, and ε is an unknown random error vector whose elements are no longer required to be independent and homogeneous.

To further develop this notion of variance modeling, assume that γ and ε are Gaussian random variables that are uncorrelated and have expectations zero and variances G and R, respectively. The variance of y is thus

+++insert Equation 10 around here +++

We can model the variance of the data, y, by specifying the structure of z, G, and R. The model matrix z is set up in the same fashion as x, the model matrix for the fixed-effects parameters. For G and R, we must select some covariance structure.

By appropriately defining the model matrices x and z, as well as the covariance structure matrices G and R, we can perform numerous mixed model analyses. The covariance structure specifies the relationship between the levels of the repeated effects. Possible covariance structures include: Unstructured, Compound Symmetry, Compound Symmetry: Heterogenous, Diagonal, Scaled Identity, etc.

4. FINDINGS AND DISCUSSION

4.1 Departure time switching behavior

The travel diary questionnaire gave us a pool of data from the 36 drivers in both experimental groups, about the journey drivers made from their own homes to the gathering places and we use it to analyze departure time switching behavior.

The purpose of estimating a logit model, which dependant variable is the decision of switching departure time, was to explore this choice in order to find out factors affecting it. We explicitly asked drivers to declare whether they changed their departure time intentionally, therefore we did not have to guess how much difference (in time) there should be in order to consider it an actual switch in departure time. This type of decision would be considered in the pre-trip stage and we started our analysis with the premise that pre-trip decisions are mainly based on the previous day experience and hence, independent of current network conditions (Jhal et al., 1998).

In Tables 5 and 6, the variables used to explain the departure time switching choice are presented. We used stepwise methods to find the best fit for the data. A critical aspect of using the stepwise logistic regression is the selection of the alpha levels. Seven runs of the SPSS logistic regression procedure were executed to check the effect of the alpha levels on both stepwise approaches (i.e. forward and backward). Results are summarized in Table 7 from which we can see that the best fit for the data was obtained when we used all variables in the model because this estimation had the lowest -2 Log Likelihood value, the highest Chi-Square value, and the highest percentage of hit.

+++insert Table 5, Table 6 and Table7 around here +++

Table 8 shows us the estimated ‘best’ model. Significant variables at the 5% level were day and risk attitude. Whether a driver had an intended route before departing, presence of congestion on previous day and gender were found to be significant at the 10% level. Day represents the experience that a driver acquired through each day of the experiment; its parameter is negative, which means that as drivers acquired experience in their trips they were finding more adequate departure time choices and this reduced switching along the days. Drivers who decided a route before departing were more likely to switch departure time; when a driver decides on a route is because he knows it somehow, including usual traffic conditions on the route, this enables drivers to switch departure time with more confidence. When congestion was found on the way the previous day, drivers were more likely to switch the next day. Female subjects were found to be more likely to switch departure time than males. Finally, drivers with a risk seeking attitude were more likely to switch departure time as would be expected.

+++insert Table 8 around here +++

4.2 Departure time considering the provision of feedback

The analysis done in this part aims at finding out whether the provision of feedback information had any effect in helping drivers make better departure time choices in order to minimize their own ‘Score’ (already explained in section 3.1.1).

Figure 6 shows that drivers were minimizing their score values with the days as expected. In the first day, those who had feedback for reference obtained in average a higher score than drivers who had no feedback, this maybe explained because the feedback for the first day consisted of only two values that resulted from two trips we made some days before the experiment, rendering it not as useful as the feedback given during next days. For the three following days of the experiment, drivers who were given feedback had smaller score values than drivers who had no feedback, and there seems to be an increasing tendency along the days. However, when we conducted an analysis of variance to test whether the difference in outcomes obtained by drivers with and without feedback is significant, results showed no evidence to assert this. We think that if we had run the experiment for more days, without taking into account the results from the first day (which resulted a bit odd), perhaps we could have been able to find a significant effect from the feedback.

+++insert Figure 6 around here +++

4.3 Compliance with the en-route information advice

We focused the analysis of route choice from the viewpoint of compliance with the en-route information advice; this is whether drivers follow the route suggested by the en-route information systems or follow their own preferences or experience in order to choose their routes instead.

We are mainly interested in examining the effect that the feedback information had on drivers. Consequently, we turned our attention to individual drivers to measure the rate of compliance of each one. For this purpose we used the expression given in Equation 5.

Figure 7 shows that drivers who had feedback appeared to have higher rates of compliance compared with drivers who had no feedback. There are several factors involved in the analysis of compliance and we were also interested in investigating which of these factors affected the drivers’ rate of compliance during our experiment. Thus, we had to model the collected data. A linear mixed model including joint data from drivers with feedback and without feedback was estimated having as dependant variable the rate of compliance. In Tables 9 and 10, the variables used to explain the drivers’ rate of compliance are presented.

+++insert Figure 7 around here +++

+++insert Table 9 and Table 10 around here +++

Four different covariance structures (i.e. ID, CS, Diagonal and CSH) were tested to fit the data. Besides AIC and BIC values, the likelihood ratio test statistic was used to look for the best fit (see Table 11). Comparisons and tests indicated that the ID matrix constituted the best fit for the data. Once the ideal matrix to model the data was determined, we proceeded with a backward selection procedure of variables to find an even better fit. The resultant model (-2LL=27.649, AIC=29.649, BIC=31.499) is shown in Table 12.

+++insert Table 11 and Table 12 around here +++

Taking a look at the estimated model and the sign of the parameters we can say that the provided feedback influenced significantly in drivers in order to comply with the information. We can also notice that the inaccuracy level of the predicted travel times had an influence on drivers (although at 10% level of significance). Drivers receiving less inaccurate information tended to comply more. Another identified factors affecting driver’s rate of compliance are: Whether drivers had access to VICS, the percentage of advice suggesting a turn and finally the years of having driver license (i.e. years of driving experience). Regarding familiarity with the network, most drivers participating in this experiment were not familiar with the area and even though this could influence on drivers to pay more attention to the information rather than on their experience, this variable in the model appeared to be not significant.

5. CONCLUSIONS

This research provides an empirical study on drivers’ behavior under traffic information and provision of feedback during a field experiment. Our focus was given especially to the effect of pre-trip departure time reference information in the form of feedback on the departure time choice and the effect of feedback on drivers’ compliance with the en-route information advice. Due to the small sample size we obtained, results from our analyses must be considered with prudence, but still some of them can help us to corroborate results from other investigators obtained through the use of different methodologies and others could lead us to a better understanding of drivers’ behavior under traffic information.

5.1 About departure time choice

- The factors found to affect on drivers’ switching departure time from one day to another were: Daily experience, as drivers acquired experience in making the journey, they were found to be less likely to switch departure time choices; Selection of a route before departure, when drivers decided upon a specific route to follow before departing they were more likely to switch departure time; Gender, female drivers were more prone to switch departure time; Risk Attitude, drivers who like to seek risk were found to be more inclined to switch departure time; Traffic Condition on previous day, when drivers encountered congestion on the road the previous day, they were more likely to switch departure time the next day; Arrival Time on previous day, drivers tended to switch more departure time when they arrived too early to the destination on previous day.

- In the study, drivers who were provided feedback (departure time reference information) had slightly better outcomes choosing departure time compared with drivers who were not provided feedback, but this small difference was not found significant.

- The accomplishment of trying to minimize either travel time or the difference between their preferred and actual travel time for drivers in our experiment, depended more on their own acquired experience day by day than on the information provided in the feedback.

5.2 About compliance with en-route information

- The factors found to affect on drivers’ compliance with the en-route information advice were: Provision of feedback, drivers who had feedback as a reference showed a higher rate of compliance compared with drivers who did not have the feedback; Accuracy of the information provided, drivers who received less inaccurate information tended to comply more; Accessibility to information about routes’ traffic condition, drivers who had access to corroborate whether the shortest advised route was congested or not through VICS were more likely to comply; Percentage of advices suggesting turning, the more the information advised to turn the less drivers were willing to comply; Driving experience, drivers who had more years of having driving license were found to comply more with the information.

- Feedback had the potential to increase the compliance rate with the en-route information advice in drivers but only when it was accompanied with good information accuracy, otherwise could rather make this rate to decline.

# REFERENCES

Adler, J., McNally, M. (1994). ‘In-Laboratory experiments to investigate driver behavior under ATIS’. Transportation Research C, Vol. 2, No. 3.

Bunch, D., Bradley, M., Golob, T., Kitamura, R., Occhiuzzo, G. (1993). ‘Demand for clean-fuel vehicles in California: A discrete-choice stated preference pilot project’. Transportation Research A, Vol. 27. No. 3.

Chen, W., Jovanis, P. (2003). ‘Analysis of driver en-route guidance compliance and driver learning with ATIS using a travel simulation experiment’. Transportation Research Record 1843, Journal of the Transportation Research Board.

City and Regional Development Bureau and Building Research Institute (CRDBBRI). (2002). Urban Transport Facilities in Japan. Volume 1. Japanese Ministry of Land, Infrastructure, and Transport.

Fujii, S., Kitamura, R. (2000). ‘Anticipated travel time, information acquisition, and actual experience’. Transportation Research Record 1725, Journal of the Transportation Research Board.

Hato, E., Taniguchi, M., Sugie, Y. (1995). ‘Influence of traffic information on drivers’ route choice’. Proceedings of the 7th World Conference on Transport Research, Vol. 1.

Ministry of Land, Infrastructure and Transport: http://www.mlit.go.jp/road/ITS/

Jackson, P. (1994). ‘Behavioral responses to dynamic route guidance (DRG) systems’. PICT International Doctoral Conference.

Jan, O., Horowitz, A., Peng, Z. (1999). ‘Using GPS data to understand variations in path choice’. Transportation Research Board 78th Annual Meeting, Washington D.C.

Jhal, M., Madanat, S., Peeta, S. (1998). ‘Perception updating and day-to-day travel choice dynamics in traffic networks with information provision’. Transportation Research C, Vol. 6. No. 3.

Jou, R. (2001). ‘Modeling theimpact of pre-trip information on commuter departure time and route choice’. Transportation Research B, Vol. 35, No. 10.

Kim, H., Chon, K. (2005). ‘Modeling en-route diversion behavior under on-site traffic information’. Journal of the Eastern Society for Transportation Studies, Vol. 6.

Lappin, J. (2000). Advanced Traveler Information Service (ATIS): Who are ATIS customers?,Report for the Federal Highway Administration. Retrieved from U.S. DOT ITS Electronic Document Library: http://www.itsdocs.fhwa.dot.gov//JPODOCS/REPTS_TE//12285.pdf

Mahmassani, H., Liu, Y. (1999). ‘Dynamics of commuting decision behavior under advanced traveler information systems’. Transportation Research C, Vol. 7. No. 2.

Makoto, I. (2004). ‘Current overview of ATIS in Japan’. 11th World Congress on ITS, Nagoya.

Mehndiratta, S., Kemp, M., Peirce, S., Lappin, J. (2000). ‘Users of a regional telephone-based traveler information system: A study of TravInfo users in the San Francisco Bay Area’. Transportation Vol. 27, No.4. Kluwer Academic Publishers B.V.

Ott, L. (1988). An introduction to statistical methods and data analysis – 3rd edition. PWS-KENT Publishing Company, USA.

Peeta, S., Yu, J. (2003). ‘Hybrid model for driver route choice incorporating en-route attributes and real-time information effects’. Transportation Research Board 81st Annual Meeting, Washington D.C.

Small, K. (1982). ‘The scheduling of consumer activities: Work trips’. The American Economic Review, Vol. 72, No. 3.

Srinivasan, K, Mahmassani, H. (2000). ‘Modeling inertia and compliance mechanisms in route choice behavior under real-time information’. Transportation Research Record 1725, Journal of the Transportation Research Board.

Targa, F., Khattak, A., Yim, Y. (2003). ‘Understanding access and use of dynamic travel information’. Transportation Research Board 82nd Annual Meeting, Washington D.C.

VICS website: http://www.vics.or.jp/english/index.html

SCOREid=TravelTimeid+Penaltyid	(1)

TravelTimeid=ActualArrivalTimeid−ActualDepartureTimeid	(2)

Penaltyid=PreferredArrivalTimei−ActualArrivalTimeid	(3)

PreferredArrivalTimei=8:30am−MinutesEarlieri	(4)

Rate of Compliance=# of Compliances# of CP in Route Taken	(5)

Pi=EY=1|Xi=11+e−β0+β⋅Xi=11+e−Zi	(6)

Pi1−Pi=1+eZi1+e−Zi=eZi	(7)

Li=lnPi1−Pi=Zi=β0+β⋅Xi	(8)

y=x⋅β+z⋅γ+ε	(9)

V=z⋅G⋅z'+R	(10)

Table 1: Study Group I: Drivers switching Departure Time (N=20)

| SUB-Groups of Study | I - A | I - B |
| --- | --- | --- |
|  | With Feedback | No Feedback |
| No. of drivers | 11 | 9 |
| No. of times driving the course | 4 |  |
| Trip Purpose | Commuting to School for the first times |  |
| Trip's Time of Day | Morning Peak |  |
| Goal for drivers | Get to destination the closest possible to preferred arrival time and make the trip in the shortest possible time |  |
| Departure Time Choice | Free Choice |  |
| Departure Time Reference Information | Provided | Not provided |
| Source of DTR Information | Feedback Sheet | - |
| Type of Information | Historical (from previous day) | - |
| Route Choice | Fixed |  |

Table 2: Study Group II: Drivers switching Route (N=16)

| SUB-Groups of Study | II - A | II - B |
| --- | --- | --- |
|  | With Feedback | No Feedback |
| No. of drivers | 7 | 9 |
| No. of times driving the course | 4 |  |
| Trip Purpose | Commuting to School for the first times |  |
| Trip's Time of Day | Morning Peak |  |
| Goal for drivers | Make the trip in the shortest possible time and get to destination before 8:30 am |  |
| Departure Time Choice | Fixed (Near 7:50 am) |  |
| Route Choice | Free among a constrained network of 6 routes |  |
| No. of Checkpoints (CP) per route | 2 or 3 |  |
| En-Route Information provided at CP | Roads' congestion condition and forecasted travel time |  |
| Source of guidance info at CP | VICS (Level 3) and Forecasting Team at Saitama University respectively |  |
| Type of guidance info at CP | Current, Descriptive |  |

Table3: Feedback Sheet Type 1 summarizing outcomes of the second day of the experiment

| Driver ID | Route | Actual Departure Time | Actual Arrival Time | Preferred Arrival Time | Travel Time | Penalty | SCORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D=B-C | E=¦C-B¦ | F=D+E |
| 3 | R3 | 7:43:05 | 8:20:14 | 8:25:00 | 0:37:09 | 0:04:46 | 0:41:55 |
| 4 | R3 | 7:38:03 | 8:11:00 | 8:25:00 | 0:32:57 | 0:14:00 | 0:46:57 |
| 5 | R3 | 7:44:48 | 8:22:22 | 8:25:00 | 0:37:34 | 0:02:38 | 0:40:12 |
| 14 | R3 | 7:38:33 | 8:18:00 | 8:25:00 | 0:39:27 | 0:07:00 | 0:46:27 |
| 1 | R3 | 7:44:44 | 8:22:50 | 8:20:00 | 0:38:06 | 0:02:50 | 0:40:56 |
| 15 | R3 | 7:41:22 | 8:17:00 | 8:20:00 | 0:35:38 | 0:03:00 | 0:38:38 |
| 16 | R3 | 7:43:09 | 8:21:00 | 8:20:00 | 0:37:51 | 0:01:00 | 0:38:51 |
| 2 | R3 | 7:40:16 | 8:17:01 | 8:15:00 | 0:36:45 | 0:02:01 | 0:38:46 |
| 6 | R3 | 7:38:07 | 8:17:18 | 8:15:00 | 0:39:11 | 0:02:18 | 0:41:29 |
| 18 | R3 | 7:38:20 | 8:17:00 | 8:15:00 | 0:38:40 | 0:02:00 | 0:40:40 |
| 19 | R4 | 7:51:30 | 8:31:30 | 8:27:00 | 0:40:00 | 0:04:30 | 0:44:30 |
| 11 | R4 | 7:44:52 | 8:20:48 | 8:25:00 | 0:35:56 | 0:04:12 | 0:40:08 |
| 12 | R4 | 7:50:00 | 8:28:45 | 8:25:00 | 0:38:45 | 0:03:45 | 0:42:30 |
| 23 | R4 | 7:51:25 | 8:31:45 | 8:25:00 | 0:40:20 | 0:06:45 | 0:47:05 |
| 22 | R4 | 7:41:26 | 8:19:00 | 8:25:00 | 0:37:34 | 0:06:00 | 0:43:34 |
| 7 | R4 | 7:43:02 | 8:18:45 | 8:20:00 | 0:35:43 | 0:01:15 | 0:36:58 |
| 8 | R4 | 7:38:16 | 8:11:33 | 8:20:00 | 0:33:17 | 0:08:27 | 0:41:44 |
| 10 | R4 | 7:40:14 | 8:13:40 | 8:20:00 | 0:33:26 | 0:06:20 | 0:39:46 |
| 20 | R4 | 7:39:43 | 8:14:30 | 8:20:00 | 0:34:47 | 0:05:30 | 0:40:17 |
| 21 | R4 | 7:38:00 | 8:15:00 | 8:20:00 | 0:37:00 | 0:05:00 | 0:42:00 |

Table 4: Feedback Sheet Type 2 given to drivers before the first day of the experiment

| Driver ID | Accumulated Rate of Compliance | Average Travel Time | Ranking |
| --- | --- | --- | --- |
|  | % | (min) |  |
| 1 | 100% | 0:28:00 | 1 |
| 2 | 100% | 0:28:00 | 2 |
| 3 | 100% | 0:28:00 | 3 |
| 4 | 100% | 0:28:00 | 4 |
| 5 | 75% | 0:30:00 | 5 |
| 6 | 75% | 0:30:00 | 6 |
| 7 | 75% | 0:30:00 | 7 |
| 8 | 75% | 0:30:00 | 8 |
| 9 | 50% | 0:33:00 | 9 |
| 10 | 50% | 0:33:00 | 10 |
| 11 | 50% | 0:33:00 | 11 |
| 12 | 50% | 0:33:00 | 12 |
| 13 | 25% | 0:35:00 | 13 |
| 14 | 25% | 0:35:00 | 14 |
| 15 | 25% | 0:35:00 | 15 |
| 16 | 25% | 0:35:00 | 16 |

Table 5: Summary of explanatory categorical variables and their coding for binary logit model

| Variable | n | Categories | Frequency | Coding |
| --- | --- | --- | --- | --- |
| Actual arrival time on previous day | 96 | Exactly on preferred time | 13 |  |
| (DIFFARR) |  | less than 10 min earlier than preferred time | 42 | (1) |
|  |  | 10 or more min earlier than preferred time | 15 | (2) |
|  |  | Few minutes after preferred time | 16 | (3) |
|  |  | Few minutes before due time | 7 | (4) |
|  |  | After due time | 3 | (5) |
| Encounter congestion on the way the | 96 | No | 74 |  |
| previous day (CONPD) |  | Yes | 22 | (1) |
| Had an Intended route before | 96 | No | 38 |  |
| departing (ROUTE) |  | Yes | 58 | (1) |
| Consulted traffic information before | 96 | No | 81 |  |
| departing (CPRTI) |  | Yes | 15 | (1) |
| Ran an errand on the way before | 96 | No | 72 |  |
| getting to gathering place (ERRAND) |  | Yes | 24 | (1) |
| Frequency of traveling to the Minami- | 96 | Once or more a week | 3 |  |
| Urawa area |  | Around once a month | 18 | (1) |
| (FAM) |  | Rarely | 57 | (2) |
|  |  | Never | 18 | (3) |
| Risk Attitude | 96 | Risk Averse | 69 |  |
| (RISK) |  | Risk Seeking | 27 | (1) |
| Has had commuting experience before | 96 | No | 17 |  |
| (COMM) |  | Yes | 79 | (1) |
| Education | 96 | High School or Technical | 45 |  |
| (EDU) |  | University | 45 | (1) |
|  |  | Graduate School | 6 | (2) |
| Gender | 96 | Male | 64 |  |
| (GENDER) |  | Female | 32 | (1) |

Table 6: Summary of explanatory quantitative variables for binary logit model

| Variable | n | Min | Max | Mean | Std. Dev. |
| --- | --- | --- | --- | --- | --- |
| Average Travel Time (ATT) | 96 | 5.50 | 116.25 | 41.91 | 26.16 |
| Minutes earlier than due time driver likes to arrive (MPAT) | 96 | 0.00 | 55.00 | 13.14 | 12.17 |
| Age (AGE) | 96 | 21.00 | 51.00 | 33.00 | 9.09 |
| Years of having driver license (LICEN) | 96 | 1.25 | 30.25 | 12.11 | 8.60 |

Table 7: Summary of statistics for different alpha values and stepwise approaches

|  | All Variables | Forward α = 0.10 | Forward α = 0.15 | Forward α = 0.20 | Backward α = 0.10 | Backward α = 0.15 | Backward α = 0.20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # of Variables | 14 | 5 | 7 | 7 | 6 | 7 | 7 |
| χ2 | 43.986 | 24.031 | 38.513 | 38.513 | 36.284 | 38.567 | 38.567 |
| Df | 22 | 4 | 11 | 11 | 10 | 11 | 11 |
| Significance of χ2 | 0.004 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| -2LL | 88.931 | 108.886 | 94.405 | 94.405 | 96.633 | 94.351 | 94.351 |
| Nagelkerke R2 | 0.490 | 0.295 | 0.441 | 0.441 | 0.420 | 0.441 | 0.441 |
| % of Hit | 80.20 | 67.70 | 72.90 | 72.90 | 79.20 | 75.00 | 75.00 |

Table 8: Best binary logit model for departure time switching choice

| Variable | B | S.E. | Wald | df | Sig. |
| --- | --- | --- | --- | --- | --- |
| DAY** | -1.142 | 0.401 | 8.117 | 1 | 0.004 |
| ATT | 0.004 | 0.031 | 0.016 | 1 | 0.899 |
| MPAT | 0.002 | 0.033 | 0.003 | 1 | 0.959 |
| DIFARR |  |  | 7.021 | 5 | 0.219 |
| DIFARR(1) | -1.031 | 0.884 | 1.359 | 1 | 0.244 |
| DIFARR(2) | 0.036 | 1.254 | 0.001 | 1 | 0.977 |
| DIFARR(3) | -2.576 | 1.096 | 5.520 | 1 | 0.019 |
| DIFARR(4) | -0.652 | 1.317 | 0.245 | 1 | 0.621 |
| DIFARR(5) | -1.209 | 3.161 | 0.146 | 1 | 0.702 |
| ERRAND(1) | -0.794 | 0.778 | 1.043 | 1 | 0.307 |
| ROUTE(1)* | 1.585 | 0.814 | 3.790 | 1 | 0.052 |
| CPRTI(1) | -1.487 | 1.145 | 1.687 | 1 | 0.194 |
| CONPD(1)* | 1.658 | 0.896 | 3.427 | 1 | 0.064 |
| GEN(1)* | 1.468 | 0.832 | 3.114 | 1 | 0.078 |
| AGE | -0.001 | 0.060 | 0.000 | 1 | 0.985 |
| EDU |  |  | 1.146 | 2 | 0.564 |
| EDU(1) | -0.851 | 0.810 | 1.103 | 1 | 0.294 |
| EDU(2) | -0.438 | 1.806 | 0.059 | 1 | 0.808 |
| LICEN | 0.031 | 0.071 | 0.189 | 1 | 0.664 |
| COMM(1) | 0.500 | 0.874 | 0.327 | 1 | 0.568 |
| FAM |  |  | 1.732 | 3 | 0.630 |
| FAM(1) | -0.570 | 1.631 | 0.122 | 1 | 0.727 |
| FAM(2) | -0.970 | 2.002 | 0.235 | 1 | 0.628 |
| FAM(3) | 0.172 | 2.293 | 0.006 | 1 | 0.940 |
| RISK(1)** | 1.918 | 0.968 | 3.924 | 1 | 0.048 |
| Constant | 23.125 | 8.569 | 7.282 | 1 | 0.007 |

Table 9: Summary of explanatory quantitative variables for linear mixed model

| Variable | n | Min | Max | Mean | Std. Dev. |
| --- | --- | --- | --- | --- | --- |
| Rate of advise suggesting a turn | 64 | 0.00 | 1.00 | 0.63 | 0.32 |
| Rate of advise suggesting the highway route | 64 | 0.00 | 1.00 | 0.35 | 0.41 |
| Age | 64 | 21.00 | 48.00 | 29.25 | 7.24 |
| Years of having driver license | 64 | 1.58 | 30.25 | 9.67 | 7.67 |

Table 10: Summary of explanatory categorical variables and their coding for linear mixed model

| Variable | n | Categories | Frequency | Coding |
| --- | --- | --- | --- | --- |
| Provided with Feedback | 64 | No | 36 | (0) |
| (FEED) |  | Yes | 28 | (1) |
| Inaccuracy of forecasted travel time | 64 | Until 33% inacuracy | 10 | (1) |
| Received |  | From 34% to 66% inaccuracy | 28 | (2) |
| (CATINAC) |  | More than 67% inaccuracy | 26 | (3) |
| Intended route at departure | 64 | No intended route | 3 | (0) |
| (IROUTE) |  | Intended route | 61 | (1) |
| Experience of a late arrival | 64 | No | 43 | (0) |
| (LATE) |  | Yes | 21 | (1) |
| Access to VICS information | 64 | No | 31 | (0) |
| (VICS) |  | Yes | 33 | (1) |
| Familiarity with the network | 64 | Very familiar | 4 | (1) |
| (FAM) |  | Somewhat familiar | 28 | (2) |
|  |  | Not familiar at all | 32 | (3) |
| Has had commuting experience | 64 | No | 8 | (0) |
| (COMM) |  | Yes | 56 | (1) |
| Used to check VICS on navigation | 64 | No | 44 | (0) |
| Equipment (VICS) |  | Yes | 20 | (1) |
| Education | 60 | High School or Technician | 36 | (1) |
| (EDU) |  | University | 24 | (2) |
| Gender | 64 | Female | 16 | (0) |
| (GENDER) |  | Male | 48 | (1) |
| Risk Attitude | 64 | Risk Averse | 44 | (0) |
| (RISK) |  | Risk Seeking | 20 | (1) |

Table 11:1 Summary of statistics for different covariance matrices

| Type of Covariance Matrix --> | Identity | CS | Diagonal | CSH |
| --- | --- | --- | --- | --- |
| # of Variables in the model | 16 | 16 | 16 | 16 |
| -2 Restricted Log Likelihood | 43.043 | 40.602 | 41.954 | 37.356 |
| Akaike's Information Criterion (AIC) | 45.043 | 44.602 | 49.954 | 47.356 |
| Schwarz's Bayesian Criterion (BIC) | 46.757 | 48.029 | 56.808 | 55.924 |
| # of Parameters in the Model | 20 | 21 | 23 | 24 |
| Difference in -2LL (with Id matrix) |  | 2.442 | 1.090 | 5.687 |
| Difference in parameters (with Id matrix) |  | 1 | 3 | 4 |
| p-value of Likelihood Ratio Test |  | 0.118 | 0.780 | 0.224 |
| Conclusion from test |  | Use ID | Use ID | Use ID |

Table 12: Best linear mixed model for drivers’ rate of compliance with information advice

| (a) Type III Tests of Fixed Effects |  |  |  |  | (b) Estimates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | df1 | df2 | F | Sig. | Parameter | Estimate | S.E. | df | t | Sig. |
| Intercept | 1 | 47 | 17.879 | 0.000 | Intercept | 1.805 | 0.392 | 47 | 4.607 | 0.000 |
| FEED** | 1 | 47 | 13.868 | 0.001 | [FEED=0]** | -0.388 | 0.104 | 47 | -3.724 | 0.001 |
| CATINAC* | 2 | 47 | 2.569 | 0.087 | [FEED=1] | 0.000 | 0.000 |  |  |  |
| VICS** | 1 | 47 | 4.848 | 0.033 | [CATINAC=1]** | 0.225 | 0.106 | 47 | 2.116 | 0.040 |
| COMM | 1 | 47 | 1.754 | 0.192 | [CATINAC=2] | -0.002 | 0.066 | 47 | -0.025 | 0.980 |
| FAM | 2 | 47 | 2.337 | 0.108 | [CATINAC=3] | 0.000 | 0.000 |  |  |  |
| UVICS | 1 | 47 | 2.666 | 0.109 | [VICS=0]** | -0.150 | 0.068 | 47 | -2.202 | 0.033 |
| TURN** | 1 | 47 | 6.907 | 0.012 | [VICS=1] | 0.000 | 0.000 |  |  |  |
| HWY | 1 | 47 | 2.731 | 0.105 | [COMM=0] | 0.250 | 0.189 | 47 | 1.324 | 0.192 |
| AGE | 1 | 47 | 2.536 | 0.118 | [COMM=1] | 0.000 | 0.000 |  |  |  |
| LICEN** | 1 | 47 | 4.558 | 0.038 | [FAM=1] | -0.065 | 0.213 | 47 | -0.304 | 0.763 |
|  |  |  |  |  | [FAM=2] | -0.181 | 0.101 | 47 | -1.781 | 0.081 |
|  |  |  |  |  | [FAM=3] | 0.000 | 0.000 |  |  |  |
|  |  |  |  |  | [UVICS=0] | 0.146 | 0.090 | 47 | 1.633 | 0.109 |
|  |  |  |  |  | [UVICS=1] | 0.000 | 0.000 |  |  |  |
|  |  |  |  |  | TURN** | -0.428 | 0.163 | 47 | -2.628 | 0.012 |
|  |  |  |  |  | HWY | -0.213 | 0.129 | 47 | -1.652 | 0.105 |
|  |  |  |  |  | AGE | -0.031 | 0.020 | 47 | -1.593 | 0.118 |
|  |  |  |  |  | LICEN** | 0.037 | 0.017 | 47 | 2.135 | 0.038 |

Figure 1: Levels in which VICS information is displayed (Source: ITS Homepage from Ministry of Land, Infrastructure and Transport, http://www.mlit.go.jp/road/ITS/)

Figure 2: Conceptual model for repetitive travel decisions under information [shown in Targa et al. (2003)].

Figure 3: Drivers’ route choice behavior before and after receiving information [shown in Hato et al. (1995)]

Figure 4: Road network used for the experiment

Figure 5: Framework for drivers’ route choice behavior under traffic information considering compliance, own intention and feedback

Figure 6: Average ‘Score’ for each day of the experiment

n=7

n=9

n=9

n=8

n=11

n=9

n=11

n=11

Figure 7: Average rates of compliance for each day of the experiment