An Activity-Based Microsimulation Model of Travel Demand in the Jakarta Metropolitan Area

Sadayuki Yagi, Ph.D.

Department of Civil and Materials Engineering

University of Illinois at Chicago

U.S.A.

Phone:	1-312-413-7447

Fax:	1-312-996-2426

E-mail:	syagi1@uic.edu

Abolfazl Mohammadian, Ph.D.

Department of Civil and Materials Engineering

University of Illinois at Chicago

U.S.A.

Phone:	1-312-996-9840

Fax:	1-312-996-2426

E-mail:	kouros@uic.edu

Abstract

The goal of the study is to develop a comprehensive activity-based modeling system in the context of developing countries, providing accurate estimates which are expected to serve as better inputs for evaluation of different transportation policy scenarios.  The case study is Jakarta, Indonesia as one of the largest metropolitan areas in Asia.  The modeling system primarily adopts a tour-based structure in which the tour is used as the unit of modeling travel instead of the trip, preserving a consistency in destination, mode, and time of day across trips.

Acknowledgement

The authors would like to express their gratitude to the National Development Planning Agency of Indonesia for permitting us to use the survey data in this study.  This study was partially funded by Foundation for Advanced Studies on International Development (FASID) of Japan.

Keywords

Activity-based models, Tour-based models, Microsimulation, Jakarta Metropolitan Area, Developing countries

# Introduction

## 1.1. Background

Activity-based modeling of travel demand treats travel as being derived from the demand for activity participation.  Travel is therefore viewed in broader context of activity scheduling. It has been argued that activity-based modeling approach can offer a rich framework in which travel is analyzed as a daily pattern of behavior related to and derived from differences in lifestyles and activity participation among individuals.  As a result, travel needs to be studied in a broader context of activity scheduling.  Activity-based modeling has been discussed in the literature since the 1970s, but practical applications have been implemented only recently (Shiftan et al., 2003).  Using a one week activity diary data, van der Hoorn (1983) developed an activity-based model including a set of logit type models for the choice of an activity given a previous location, and the choice of a location, given a previous location.  Kitamura and Kermanshah (1983) developed a sequential logit-based model of out-of-home activity choice and destination choice in which choice of one activity is assumed to be conditioned upon the previous activity.  Axhausen (1990) used discrete events simulation approach to develop a model of activities and network traffic flows for Germany assuming activity chains are known and used to schedule event durations. Several other academic researches focused on developing activity-based models in late 80s and early 90s including STARCHILD (Recker and McNally, 1986), SMASH (Ettema et al., 1993), and SCHEDULER (Golledge et al., 1994).  The revolution in hardware and software capabilities and advances in simulation techniques in late 90s and the new millennium resulted in several activity-based microsimulation models including ALBATROSS (Arentze et al., 1998), CEMDAP (Bhat et al., 2003), FAMOS (Pendyala et al., 2004), and ILUTE (Salvini and Miller, 2005).

The goal of the study is to develop a comprehensive activity-based microsimulation modeling system of which structures and control factors may be significantly different in the context of developing countries, providing accurate estimates which are expected to serve as better inputs for evaluation of different transportation policy scenarios.  The study simulates the way individuals schedule their daily activities and travel in an urban region of the developing world.  The case study is the Jakarta, Indonesia as one of the largest metropolitan areas in Asia with a population over 21.6 million people and 5.6 million households.  The study aims at developing a practical model that can replicate patterns of activity-travel and test different transportation policy scenarios for the Jakarta Metropolitan Area.

## 1.2. Jakarta Metropolitan Area

In the Jakarta Metropolitan Area, a variety of urban transportation management policies are currently being examined, discussed, or implemented.  For example, the “3-in-1” scheme, in which only high-occupancy vehicles are allowed to use central arterial roads during peak periods, has recently been more widely and strictly enforced along with the introduction of a new bus rapid transit (BRT) using the centermost lane of the arterial roads as a dedicated “busway.”  More new policies such as area pricing and license plate restriction are now being discussed for implementation as well as extension of the BRT system.  In addition, raising the fuel price is a current controversial issue that is being considered and implemented by the Indonesian government.  As such, it is hoped that the proposed new models will contribute to the improvement of the methodology of travel demand forecasting and evaluation of urban transportation policy scenarios in the region.  This paper describes the general framework and methodology of activity-based model that has been applied in this study along with the results of model estimation.

In “The Study on Integrated Transportation Master Plan (SITRAMP)” conducted in the Jakarta Metropolitan Area from November 2001 to March 2004 (National Development Planning Agency 2004), detailed transportation surveys such as Household Travel Survey (HTS) and analyses were undertaken to prepare a comprehensive long-term transportation plan.

The primary objective of these surveys was to develop and calibrate state-of-the-art disaggregate travel demand models to simulate present and future interactions between socioeconomic distribution and transportation demand.  Among a variety of the surveys conducted, the HTS, which is a large scale home interview survey of household daily travel, provides the largest and most comprehensive travel data in the region.  The dataset covers as many as 166,000 households equalling three percent of the population and provides daily travel patterns on a weekday and detailed information on household socio-demographic characteristics.  The activity diary survey (ADS) provides a detailed four-day diary covering around 4,000 individuals.  The large datasets obtained for this study provide a unique opportunity to conduct numerous other research works.

## 1.3. Relationship between HTS and ADS

It is worth noting that the activity-based model requires a much richer and more detailed dataset than those required for the conventional travel demand modeling systems.  Fortunately, the Jakarta Metropolitan Area has excellent sources of survey data including a regular full-scale household travel survey (HTS) and an activity diary survey (ADS).  Both surveys cover activity-travel information of individuals of all generations including children who are 5 years or older (Yagi and Mohammadian, 2005).  The activity-based modeling system is developed using these two datasets.

The ADS was conducted originally to support the result of the HTS, providing a series of daily household activities on the basis of every 15 minutes for 24 hours over four consecutive days including both weekdays and weekends.  As such, it involves data collection of a series of daily activities, including type, start- and end-time of the activities, and the modes used.  Since all the activities performed both out of home and at home over each time period are recorded continuously as well as modes of travel, trips which tend to be overlooked in the regular HTS are also captured.

Samples for the ADS were taken from 50 villages (called kelurahan or desa), and for each village 24 responding households were randomly selected from the HTS samples.  Both household travel and activity diary surveys were conducted in 2002, only a few months apart.  Thus, the ADS samples are linked to the corresponding observation in the HTS by a unique identifier, and the detailed socioeconomic attributes of the samples for the activity diary survey were collected from the HTS database, which include income level, car/motorcycle ownership, occupation, work/school address, etc.  For this study, effective data for only weekdays were collected, resulting in 7,690 person-day ADS effective data records throughout the Jakarta Metropolitan Area.

# ACTIVITY-BASED MODELING STRUCTURE

## 2.1. Definitions

A “trip” is defined as a travel between two activities representing the trip purpose (Home to Work, Home to School, etc).  The term “purpose” in this study is used to present the activity performed at the trip end.  Furthermore, each trip record is coded with travel mode (auto, motorcycle, transit, etc.).  A “tour”, on the other hand, is defined as a chain of trips which start from a base and return to the same base.  One or more activities (i.e., purposes) are involved in the course of a tour.  In order to analyze daily activity-travel tour patterns in this study, a tour has been considered a home-based tour if it starts from home and ends at home.

As for out-of-home activities, they are often grouped into the following three commonly categorized activity types (Vovsha et al., 2004a):

•	Mandatory activities (e.g., work, university, or school),

•	Maintenance activities (e.g., shopping, banking, visiting doctor, etc.), and

•	Discretionary activities (e.g., social and recreational activities, eating out, etc.).

For this study, mandatory activities are further divided into work and school purposes because there is essentially a difference between work and school as to by whom and when such activities are carried out.  The last two activity types, maintenance and discretionary activities, are often treated as one activity type, that is, non-mandatory pattern (Bradley and Vovsha, 2005) that can be distinguished from mandatory primary activities such as work and school patterns.  In this study, this classification has been applied to deal with intra-household interactions in the activity-based microsimulation.  In addition, it is worth noting that in-home activities (e.g., working at home, shopping online, taking care of a child/old person, being sick, etc.) deserve special consideration though there is not much research done with regard to trade-offs between in-home and out-of-home activities.

## 2.2. Modeling Framework

Bowman and Ben-Akiva (2000) developed a model in which they consider the daily activity-travel pattern as a set of tours.  Home-based tours, which are defined as the travel from home to one or more activity locations and back home again, are subdivided into “primary” and “secondary” tours based on activity priority.  Activities are prioritized based on the purpose of the activity, with work activities having the highest priority, followed by work-related, school, and all other purposes.  Within a particular purpose, activities with longer durations are assigned higher priorities.  The tour of the day with the highest priority activity (i.e., the “primary” activity) is designated as the “primary” tour and others are designated as “secondary” tours.

For this study, the fundamental modeling approach is a discrete choice model based on the random utility maximizing principles.  It has been shown that the multinomial logit model is the most popular form of discrete choice model in practical applications (Mohammadian and Doherty 2005).  Nested logit model, which has also been utilized in this study, is a model that has been developed in order to overcome the so-called independence of irrelevant alternatives (IIA) limitation in the multinomial model by modifying the choice structure into multiple tiers.  Nested logit models are very commonly used for modeling mode choice, permitting covariance in random components among nests of alternatives.  Alternatives in a nest exhibit an identical degree of increased sensitivity relative to alternatives in the nest (McFadden 1978).  A nested logit model has a logsum or expected maximum utility associated with the lower-tier decision process.  The parameter of the logsum determines the correlation in unobserved components among alternatives in the nest (Daganzo and Kusnic 1993).  The range of this parameter should be between 0 and 1 for all nests if the nested logit model is to remain globally consistent with the random utility maximizing principle.

As such, the modeling system in this study primarily adopts a modified version of the frameworks proposed for Boston (Bowman and Ben-Akiva, 2000), and Portland (Bowman et al., 1998; Bradley et al., 1999) to develop an uncomplicated yet credible model which can replicate the patterns of activity and travel in developing world.  It has a similar tour-based structure in which the tour is used as the unit of modeling travel instead of the trip, preserving a consistency in destination, mode, and time of day across trips.  The overall modeling structure is depicted in Figure 1.  All models are for home-based tours unless otherwise specified.  It is a system of random utility based disaggregate logit and nested logit models assuming a hierarchy of model components, with three types of major models, namely, choices of daily activity-travel patterns, times of day, and mode and destination in the hierarchy.  Lower level choices depend on the decisions at the higher level, and higher level decisions are linked to the lower level choices through the logsum variables reflecting expected maximum composite utility of lower-level choices.  Furthermore, two types of additional sub-models are added to this framework in order to determine additional characteristics of tours, that is, mode and destination choice for work-based sub-tours and location choice of intermediate stops.  Each model is described in the subsequent sections.

As a basic input to the proposed activity-based modeling system, various household and household member information, zone-based socioeconomic and land use data, and highway and transit network-based data are prepared, and the modeling system will generate people’s daily activity-travel patterns, tours, and trips that can be integrated into origin-destination (OD) trips by mode and by time of day for full network assignment.  The base year is set as 2002 and all the models are estimated based on the input as of 2002.  For future years, the population will be updated first to prepare for household and household member information; especially, a household auto/motorcycle ownership choice model is adopted to set the number of autos and motorcycles owned by each household.

The modeling system is developed using the available ADS and HTS data from the Jakarta Metropolitan Area.  As one can imagine, the models are quite dependent on individual and household attributes, as well as habits, customs, preferences and decision-making processes.  In Jakarta, these attributes are expected to be quite different from those observed in the U.S. or other developed world.  Although the basic modeling framework is applicable anywhere, detailed modeling structure as well as explanatory variables may also be different from the original US-based models in the context of a case study for a developing country.

# Model Description

## 3.1. Daily Activity-Travel Pattern Choice

Daily activity-travel patterns (DAPs) except for home, which means staying at home all day, are defined by primary activity, primary tour type, and number and type of secondary tours.  The most important out-of-home activity of the day that often has the longest duration is designated the primary tour and all other tours are designated as secondary.  All activities in a tour are ranked by priority, with work and work-related, school, maintenance (e.g., shopping, banking, visiting doctor, etc.), and discretionary (e.g., social and recreational activities, eating out, etc.) purposes, in this order.  Thus, primary activities or purposes are classified as home (H), work (W), school (S), maintenance (M), and discretionary (D) for the sake of modeling.

Both primary and secondary tours are treated as home-based.  Primary tour type is defined by presence and sequences of intermediate stops and presence of (work-based) sub-tours in a tour.  As presented in Table 1, the primary tour type classification depends on whether it is a simple tour from home to a destination and then back to home again with no intermediate stops (i.e., Codes 1, 5, 9, 13, 17, 21), or a tour with at least one intermediate stop for another activity on the way from home (i.e., Codes 2, 6, 10, 14, 18, 22) or on the way back home (i.e., Codes 3, 7, 11, 15, 19, 23); and whether it includes at least one sub-tour (for work tours only) which is further divided into an intermediate returning-home sub-tour (i.e., Codes 9-12) and a sub-tour to somewhere else (i.e., Codes 5-8).  Each classification is represented as a string of letters indicating a sequence of activities.  Note that an ‘H’ is included at the beginning and the end of each string to show that it is a home-based tour.  For simplicity, activity purpose of intermediate stops and work-based sub-tours (except for intermediate return home) is not considered in this model, and those are represented as an ‘O’ in the activity sequence.

On the other hand, secondary tour type is defined by activity purpose, and it is classified into maintenance and discretionary activities.  Number of secondary tours is counted by activity and classified into 0 and 1 or more secondary maintenance tours, and 0, 1, and 2 or more secondary discretionary tours as shown in Table 2.  Combination of the primary activity, primary tour type, and number and type of secondary tours brings about a choice set of 121 DAP alternatives, including 1 home pattern, 60 primary work tour patterns, 20 primary school tour patterns, 20 primary maintenance tour patterns, and 20 primary discretionary tour patterns.

The most notable difference in relative frequency of the DAP alternatives between the HTS and ADS datasets may be the relative frequency of the home pattern.  In the HTS, the percentage of staying at home all day is as high as 22 percent, while the one derived from the ADS is only about 6 percent.  In addition, alternatives of simple primary tour type with no secondary tours have higher frequencies in the HTS.  The main cause for such differences may be that relatively short-distance tours, especially those made by non-motorized mode of transport tend to be overlooked in the HTS.  Although modeling based on the HTS database could eventually have no major impact on the forecast of longer-distance motorized trips, the ADS database has been selected for model estimation of DAPs.

Efforts were made to include as many DAP alternatives as possible for model estimation.  In practice, however, at least 15-20 samples were necessary for each alternative to derive significant estimates from the model.  Looking into 7,690 weekday samples in the ADS dataset, we have combined primary tour types of some alternatives with low frequencies as well as some secondary tour types and numbers, and have included 35 alternatives in the model for estimation.

In the microsimulation, DAPs that are generated under each of these ‘representative’ alternatives will be further given more details as to the actual primary tour type and the fixed numbers of secondary maintenance and discretionary tours based on the probabilities derived from the ADS sample distributions.  In addition, neither number nor sequences of intermediate stops in a secondary tour are considered in the model.  Secondary tours should be properly classified into several additional tour types related to presence and sequences of intermediate stops.  Instead of using logit choice models, classifications that correspond to Codes 17 to 24 in Table 1 will be given to the secondary tours through the look-up tables created from the observations in the ADS.

The model for daily activity-travel pattern choice, which is placed at the top of the entire modeling system, has a two-tier nested logit structure, with a choice of whether to go out of home and travel or stay at home all day in the upper tier and a choice of out-of-home DAP alternatives defined by primary tour activity, primary tour type, and number and type of secondary tours in the lower tier under the out-of-home.  On the other hand, home alternative is a degenerate branch with only one stay-home-all-day alternative.  The modeling structure of DAPs is depicted in Figure 2.  This model is estimated using many variables explaining attributes of the household and the individual.  Logsum variables from the lower level, that is, time-of-day choice for primary work, school, maintenance, and discretionary tours, and secondary maintenance and discretionary tours are also included in the model, where applicable.

## 3.2. Time-of-Day Choice

In order to model the time-of-day (TOD) choice, a day is divided into five time periods, namely, early morning (“EM”, 3:00 – 6:29), a.m. peak (“AM”, 6:30 – 9:59), midday (“MD”, 10:00 – 15:59), p.m. peak (“PM”, 16:00 – 18:59), and night (“NT”, 19:00 – 2:59).  These five time periods are distinguished considering not only characteristic hourly traffic volume but also the operation hours of Jakarta’s unique “3-in-1” traffic regulation (i.e., morning operation from 6:30 to 10:00 as of 2002, and evening operation from 16:00 to 19:00 added since 2004).  Alternatives are created by combining the time period to leave home to start the tour and the time period to leave the destination of the main activity to start the returning segment of the tour.  Frequencies of tours starting early in the morning (EM) are relatively high in Indonesia; according the ADS, over 90 percent of people get up by 6:00 a.m. on weekdays.  Assuming for simplicity that there are no tours that last over night, 15 TOD combinations or alternatives are identified.

As shown in Figure 3, the TOD choice is a multinomial logit model with 15 alternatives, and it is estimated separately for each purpose (i.e., work, school, maintenance, and discretionary).  Note that students’ TOD choice should be significantly different from that of workers because schools in Indonesia usually have a half-day shift system (i.e., two or three shifts per day).  The above four models are to generate logsums that will be passed to the single, upper-level DAP choice model.  Since model estimation by purpose needs enough observations for each of the 15 TOD alternatives, samples have been taken from the HTS database.  For each purpose, a total of around 25,000 tour samples were used for modeling, and we have ensured that the sample households and individuals that were used for estimation of the DAP choice model are included in the dataset for TOD choice modeling.

## 3.3. Mode and Destination Choice

Mode and destination choice is placed at the “bottom” of the hierarchy consisting of the three major models, and is conditional on decisions at the higher levels, that is, choices of DAPs and TODs by purpose.  As is the case with the TOD choice model, the mode and destination choice is modeled by purpose.  The results of the HTS show that at least over 90 percent of people return home using the same mode as they used for the from-home trips, though the percentages vary depending on modes and purposes.  This suggests that from-home trips constrain the modes and destinations of the subsequent segments such as returning-home trips.  Therefore, for mode and destination choice, from-home trips are focused on and used to estimate the entire tour mode and destination choice model, because these trips constrain the modes and destinations of the subsequent segments such as returning-home trips.

Eight most commonly used combinations of travel modes observed in the region are considered.  These include auto drive alone (ATD), auto shared ride (ATS), motorcycle (MTC), taxi (TXA), motorcycle taxi (TXM), transit with motorized access (TRM), transit with non-motorized access (TRN), and non-motorized transport (NMT).  Although auto drive alone and shared ride treated as a single alternative in the previous SITRAMP study, we have clearly distinguished these two alternatives in order to make the model more sensitive to the transportation policies, especially those related to high-occupancy vehicles.  Motorcycle taxi is a unique mode of transport but is quite common in urban areas of the developing world.  It usually serves relatively shorter-distance trips using any types of roads from alleys to arterials, especially in cases where autos, taxis, or buses are hardly available.  Transit has been divided into two, that is, transit with and without motorized access.  The former includes park-and-ride or kiss-and-ride access by private auto or motorcycle; however, access by the above-mentioned motorcycle taxi is more common in the Jakarta Metropolitan Area.  As for non-motorized transport, walking is a dominant mode though bicycles and pedicabs are also observed in some suburban areas.

As for the destination choice, 11 representative destinations are considered for each tour in order to reduce the computational burden.  These destinations are sampled from the 336 traffic analysis zones (TAZs) using the stratified importance sampling method.  For each purpose, the strata of destinations are constructed based on the distance as well as a size variable which indicates the magnitude of attraction in the destination (Bradley et al., 1998).  Size variables have been set as total jobs for work, total students at school place for school, total service industry jobs for maintenance, and the sum of service industry jobs and households for discretionary activities.  As a result, this sampling method leads to higher probabilities of being selected for zones closer to the origin (i.e., home) as well as for zones with larger potential of corresponding attraction.

Actual sampling strata for these 11 representative destination zones are as follows:

•	Zone 1, “sampled” from the origin (home) zone;

•	Zones 2 and 3, sampled from a distance less than D1;

•	Zones 4 and 5, sampled from a distance between D1 and D2 and total jobs smaller than J;

•	Zones 6 and 7, sampled from a distance between D1 and D2 and total jobs greater than J;

•	Zones 8 and 9, sampled from a distance greater than D2 and total jobs smaller than J;

•	Zones 10 and 11, sampled from a distance greater than D2 and total jobs greater than J,

where:

•	D1 and D2 are the 20th and 60th percentile distances from the origin zone to all other tour destinations for each purpose, respectively; and

•	J is the 50th percentile size variable of all tour destinations for each purpose.

While the value of size variable, J, stays the same regardless of the origin zones, the values of distance, D1 and D2, are different depending on the origin zone.  Hence, the composition of the above sampling strata for destination choice also differs by the origin zone.

As this is a joint model of mode and destination choice, total number of choice alternatives is presumed to be 88 (i.e., total number of modes multiplied by total number of destination zones).  However, frequencies of each mode in relation to the destination strata were investigated.  Due to the extremely low frequencies observed from the dataset, it is assumed that motorcycle taxi alternatives are unavailable if the distance to the destination zone is greater than D2, and non-motorized transport alternatives are unavailable if the distance is greater than D1.  Thus, the maximum number of available alternatives is reduced to 76.  Auto (drive alone) alternatives are made unavailable for individuals under 17 (i.e., pre-driving age) and for those who do not have access to any automobiles as indicated in the survey.  Additionally, motorcycle alternatives are made unavailable for those who do not have access to any motorcycles.

The model has a two-tier nested logit structure.  As shown in Figure 4, for each representative zone, auto drive alone, auto shared ride, and motorcycle; and taxi, motorcycle taxi, transit with motorized access, and transit with non-motorized access are each placed in the second tier under different nests while non-motorized transport is placed as a degenerate branch.  Although nests are created for each representative destination zone, logsum parameters are set to be common for the nests which involve the same mode group.  The model is estimated separately for each purpose (i.e., work, school, maintenance, and discretionary).  Each of these four models will generate a logsum that will be passed to the TOD choice model of the same purpose.  Samples have been taken from the HTS database; in fact, each model contains samples of the same 25,000 tours that were used for modeling TOD choice.  That is, this dataset again includes the sample households and individuals that were used for estimation of the DAP choice model.

## 3.4. Additional Sub-Models

Furthermore, two more sub-models are included in the activity-based modeling system, namely, mode and destination choice for work-based sub-tours (in work tours only) and intermediate stop location choice.

### 3.4.1. Mode and Destination Choice for Work-Based Sub-Tours

For the work-based sub-tour mode and destination choice, the methodology is similar to that of the above-mentioned home-based tour mode and destination choice.  In the same way, the alternatives consist of combinations of eight travel modes and 11 representative sub-tour destinations.  The origin zone to set the sampling strata for this model is a zone in which the workplace is located, that is, the destination zone of the corresponding home-based work tour.  The size variable which determines the strata of sub-tour destinations has been set as total jobs, just like for home-based work tours.  For simplicity, only one model is estimated regardless of the actual activity conducted for the work-based sub-tour.

### 3.4.2. Intermediate Stop Location Choice

This model determines only the location of intermediate stops made on the way from home or on the way back home.  As for the mode, it is fixed to be the same as the mode that was determined in the upper, home-based tour mode and destination choice model, and switching modes before and after the stop is not considered.  For intermediate stop location choice, the stratified importance sampling method that was used for the mode and destination choice models is adopted again to set 11 representative zones for stops.  However, the methodology has been slightly modified; that is, both home zone and destination zone of the tour are regarded as the origin zone to set the sampling strata for this model.  If a zone is classified into different representative zone strata depending on the home and destination zones as the origin, the stratum that is closer to the origin is chosen as a rule.

# MODEL Development

## 4.1. Model Estimation Result

Major characteristic explanatory variables were described under each model.  In addition, other socioeconomic variables related to households and household members are introduced to the utility functions, including household composition (i.e., number of members, adults, children, and infants); household income; vehicle ownership (i.e., number of automobiles and motorcycles owned by the household); household location (i.e., central business district (CBD), Jakarta city, and urban/suburban area); status of the individual; income of the individual; school type for student; and gender and age of the individual.

Parameters of the models are actually estimated from lower to upper models in this order to generate and pass logsums to the upper models, while the model estimation results are described first from the top model.  For nested logit models, parameters are estimated simultaneously across the two tiers using full information maximum likelihood estimation (FIML) approach.

### 4.1.1. Daily Activity-Travel Pattern Choice

The estimated nested logit model is presented in Table 3.  The adjusted rho-squared value, as a measure of fit of the model, is 0.50, showing a pretty good model fit.  The logsum coefficient capturing the effect of the expected maximum utility from out-of-home activity patterns is estimated as 0.81, and coefficients of the logsums taken externally from the lower time-of-day choice models are estimated as 0.13 – 0.44.  These coefficients all fall within the theoretically acceptable range between 0 and 1 in a nested logit structure with significant t-values.  Modeling outcomes are summarized and discussed below.

Household size is relatively large in Jakarta and there are a variety of composition patterns of household members.  Results presented in the table suggest that some variables regarding the household composition are significant in the choice of daily activity-travel patterns (DAPs), reflecting interactions among household members.  For example, large household size implies that there are more (young or old) members to be taken care of in the household, thus increasing the utility of having a work tour pattern with an intermediate home return.  Similarly, as the number of children increases, people tend to hurry home without any stops on the way back.  It is also obvious that a household member with children has greater utility of having school patterns.  In the case of only one adult with children in the household, he or she is more likely to have a work-based sub-tour to save some time on the way, and is more likely to have secondary discretionary tours probably with the children.  On the other hand, as the number of adults increases, allocation of maintenance activities per adult becomes smaller, thus reducing the likelihood of having primary and secondary maintenance tours.

Household income has an influence in many dimensions of activity-based models of the Jakarta Metropolitan Area.  Major influence of the household income on the DAP choice is related to work tours.  Modeling results show that people in the lower-income household have greater utility of work tours.  Though further investigation is necessary, this may be because higher-income workers tend to have less work hours, more days off, or more chance to work at home (i.e., telecommuting).  Similar implications can be derived from the fact that higher-income people are more likely to have intermediates stops in primary discretionary tours.

As for mobility, it is more or less limited in the Jakarta Metropolitan Area, and, in this sense, auto availability greatly enhances mobility of people, reducing the likelihood of staying at home all day.  Though number of autos could be a significant variable in the mode choice model as well, it proved to be a key factor in the DAP choice as well.  Additionally, it increases the utility of having intermediate stops in primary maintenance tours.  Household location also has an influence of mobility.  Modeling results suggest that if the household residence is located in Jakarta, the utility of having work tours with an intermediate home return or having primary tours with stops on the way from home is expected to increase because of generally shorter travel time and distance, or convenience.  It also increases students’ utility of having stops on the way in school tours.

Differences in gender and/or age seem to be significant factors for DAP choice modeling in Jakarta.  Males or male adults have greater opportunity to travel, increasing the utilities of all tours except for maintenance, as well as increasing the utilities of having intermediate stops.  Male adults reduce the utility only for secondary maintenance tours.  Meanwhile, female adults are less likely to have school tours probably because of the fact that the ratio of females (47.3 percent) to all university students is smaller than that of males (52.7 percent).  Age also has an influence on activity patterns.  Modeling results imply that older people (of age over 65) have a greater utility of staying at home all day; moreover, they tend to have primary discretionary tours or maintenance tours rather than to have work tours.  On the other hand, younger people (of age under 20 or 25) have a disutility for some complex work tour patterns with sub-tours.  Such tendencies may match with the economically active age (i.e., 18-60).

Since all household members of age 5 or older are included for modeling, there is a diversity of status of individuals.  Some of those variables are quite reasonable.  For example, unemployed people are more likely to stay at home all day or to have primary discretionary tours.  Full-time workers tend to have more work-based sub-tours.  Being a homemaker increases the utility of primary maintenance tours.  As for schoolchildren, especially those of pre-driving age have rarely been included in activity-based models in the U.S.  For the Jakarta Metropolitan Area, students are included in the models along with variables indicating school types.  To summarize, younger schoolchildren tend to have secondary maintenance or discretionary tours rather than to have school tours with intermediate stops, while students of higher education are more likely to have intermediate stops.  Thus, a variety of variables regarding status greatly helped to better estimate the DAP choice model for all generations.

### 4.1.2. Time-of-Day Choice

The estimated multinomial logit model is presented in Table 4.  The adjusted rho- squared value, as a measure of fit of the model, depends on the tour purposes and ranges from 0.30 to 0.57, showing a good model fit.  The external logsum coefficients capturing the effect of the expected maximum utility from the lower mode and destination choice models also depend on the tour purposes and are estimated as 0.61 – 0.96.  These coefficients all fall within the theoretically acceptable range between 0 and 1 with significant t-values.  Modeling outcomes are summarized and discussed below.

Variables indicating tour types have been successfully included in the time-of-day (TOD) choice models.  They are conditional on DAPs and therefore important to generate utility logsums for the upper DAP choice.  Having intermediate stops, especially on the way back home, generally reduces the utilities of choosing late returning departures such as p.m. peak and night.  Secondary tours, which exist only in maintenance or discretionary tours, increase the utilities of rather late tour starts like in the midday or later.  In particular, secondary tours with primary work tours reduce the utilities of TOD combinations that take up significant part of the daytime; furthermore, secondary maintenance tours with primary work tours reduce the utilities of TOD combinations that take up evening and night as well.

Some variables reflecting interactions among household members are also included in the TOD choice.  In general, those with children or infants in the household have a greater utility for relatively shorter-duration TOD combinations during the daytime.  In particular, they are more likely to have maintenance tours earlier in the day while they are less likely to have school or maintenance tours later in the day.  Meanwhile, a person of a one-member household tends to have a maintenance tour in the night time.

Income has a great influence also on the TOD choice.  For work tours, personal income which is more directly related to a worker’s job type also has a great influence as well, and the modeling results imply that lower-income people tend to have work tours which either start early in the morning or return home in the night time.  For school tours, those of higher-income households are more likely to have longer-duration school tours.  For maintenance and discretionary tours, TODs for those activities seem to be different depending on the household income; lower-income people have greater utilities of these activities earlier in the day while higher-income people have greater utilities later in the day.

Variables regarding gender and age are again playing significant roles in the TOD choice.  Males increase the utilities for work tours of longer duration while older people increase the utilities for work tours of shorter duration.  Though gender difference seems to have a less impact on the TOD choice for school tours, age seems to have some impact implying that students of higher-education increase utilities for a broader variety of TOD combinations of school tours, especially those of longer duration.  As for maintenance and discretionary tours, males or younger people are more likely to choose combinations of TODs toward the night time.

Implications of some of the status-related variables included in the models are quite straightforward.  For example, full-time workers tend to choose longer-duration TOD combinations for work tours and choose shorter-duration TOD combinations either in the early morning or in the night time for maintenance or discretionary tours.  Homemakers tend to choose maintenance or discretionary tours that start and return during the daytime periods.  Furthermore, some variables indicating job or workplace types have proved to be significant in the TOD choice for work tours, such as commercial occupations/workplaces that increase the utilities of early starts or late returns, and agricultural occupations/workplaces that increase the utilities of early starts and midday returns.  As for school tours, relatively a variety of TOD combinations have been observed.  This may be because of the shift system (two or three shifts per day) adopted for elementary and high schools in Indonesia.

### 4.1.3. Mode and Destination Choice

Results of the home-based mode and destination choice models are presented in Table 5.  The model by purpose shows a good fit with the adjusted rho-squared value ranging from 0.38 to 0.62.  The log-sum parameters from the lower branches range from 0.66 to 0.79 for private modes and from 0.50 to 0.76 for taxi and transit modes, staying within a reasonable range.  The external logsum coefficient capturing the effect of the expected maximum utility from the lower work-based sub-tour mode and destination choice model has been estimated as 0.10 in the work tour model.  Coefficients of the external logsums from the lower intermediate stop choice models depend on the tour purposes and are estimated as 0.04 – 0.09.  These coefficients all fall within the theoretically acceptable range between 0 and 1 with significant t-values.  Modeling outcomes are summarized and discussed below.

Among several types of cost and time-related variables, so-called generalized travel time proved to work best in the model.  It is computed from the preliminary network assignment highway or transit network by origin-destination zone pair and by mode, including not only travel times (in-vehicle time and waiting time in the case of transit) but also times that have been converted from monetary costs such as transit fares and highway tolls.  Three kinds of time values are used to calculate the generalized time.  These are for low, middle, and high income households, which were estimated through a stated preference opinion survey conducted in the SITRAMP study.  In particular, for work and school tours, coefficients of the generalized time are estimated separately for auto, motorcycle, taxi, and motorcycle taxi and transit.  While the coefficients for auto and taxi have higher absolute values, the coefficient for motorcycle shows the lowest sensitivity to the generalized time.  This result seems reasonable because auto and taxi are generally used by middle to high-income people and motorcycle is used by low to middle-income people, as indicated by other income-related variables included in each mode.  On the other hand, the coefficient for motorcycle taxi and transit stays somewhere in between, and relatively few income-related variables are included in the utility functions.

The generalized travel time also works as one of the variables that determine the utilities for destination choice.  So does natural logarithm of the size variable (i.e., total jobs for work, total students at school place for school, total service industry jobs for maintenance, and the sum of service industry jobs and households for discretionary activities).  As for other destination-related variables, the origin zone dummy has a very high t-value across all tour purposes, increasing the utility for intra-zonal tours.  Above all, the parameter in the discretionary tour has the highest value (i.e., the greatest increase in the utility of intra-zonal tours).  This may coincide with the fact that discretionary tours has the highest intra-zonal tour ratio of all purposes observed in the HTS (i.e., 42.4 percent for work, 68.7 percent for school, 68.5 percent for maintenance, and 80.9 percent for discretionary).  Dummy variables identifying zones inside the urban area have also been included in school, maintenance, and discretionary tours.  Other variables included in the mode and destination choice are densities of jobs, service jobs, students (at school place), and households by zone.  In addition, fractions of land for business and recreational use have been included in the models.

As a whole, the models have captured the key significant variables, including not only the above- mentioned trip and zone attributes, but also tour or activity-related variables such as presence of intermediate stops, distinction of primary or secondary tours, and start times of the tour or returning segment of the tour.  To put it briefly, presence of intermediate stops in a tour increases the utilities of private modes including taxis that are more convenient for making stops.  Secondary tours increase the utilities of motorcycles and non-motorized transport instead of transit or taxis, implying that travel distance is generally shorter in secondary tours.  Tours with a home return in the night time increase the utilities of private modes including taxis, probably because of convenience and security.

Furthermore, various socioeconomic attributes of both households and individuals also play significant roles in the models.  Focusing on comparison with similar studies conducted for the U.S. cases (Bradley, et al. 1999), the modeling results give the following characteristic implications in the context of Jakarta.

•	Car competition between household members, which often has an influence on mode choice in the U.S., is not an “issue” in the case of Jakarta.  The auto ownership ratio is still low in Jakarta, and whether the household owns an auto or motorcycle is more significant for mode choice rather than actual number of vehicles owned (except for school tours).

•	In Jakarta, auto shared ride more often indicates those who do not actually drive but have chauffeurs.  Carpools or vanpools do exist in Jakarta, but those are more commercially operated and treated as unofficial transit.  As a result, auto shared ride is characterized rather as a mode for high-income people.

•	Income has a greater influence on mode choice.  Generally, utilities of auto and taxi increase as the income becomes higher, while utilities of motorcycle, transit (with non- motorized access), and non-motorized transport increase as the income becomes lower.

•	Gender and age also play more active and distinct roles in the models.  That is, males have greater utilities of private modes, while females have greater utilities of taxi and transit.  In addition, older people have greater utilities of private modes including taxis rather than non-motorized transport or transit (except for school tours).

•	Some variables indicating the status of individuals directly increase the utilities of certain travel modes.  For example, homemakers have greater utilities of non-motorized transport for maintenance and discretionary tours.  Full-time workers have greater utilities of autos or motorcycles for discretionary tours.

•	A variety of commuting allowances are commonly provided by the employer in the case of the Jakarta.  As such, availability of such allowance mainly increases utilities of private modes in work tours.

### 4.1.4. Additional Sub-Models

#### Mode and Destination Choice for Work-Based Sub-Tours

The estimated work-based sub-tour mode and destination model is presented in Table 6.  The adjusted rho-squared value, as a measure of fit of the model, is 0.72, showing a very good model fit.  The log-sum parameters from the lower branches are estimated as 0.90 and 0.98 for private modes and taxi and transit modes respectively, staying within a reasonable range with significant t-values.  Modeling outcomes are summarized and discussed below.

In this model, generalized travel time also proved to work best among several types of cost and time-related variables.  The coefficient estimated as generic across the travel modes, however, has a smaller magnitude compared to those in the home-based tour mode and destination choice.  This may imply that selection of a sub-tour destination does not depend on the cost or time as much as the destination choice for home-based tours.  On the other hand, the origin zone dummy has a very high t-value in this model as well, increasing the utility for intra-zonal tours.  Other zone-related variables included in the model are the urban area zone dummy, the service job density, and the fraction of land for business use.

For mode choice, a dummy variable indicating whether the same mode was used to commute between home and workplace has been included in the model with highly significant t-values except for non-motorized transport.  As such, the mode choice for work-based sub-tours is closely related to the main tour travel mode.  As for other variables, tour-related variables such as start times of the tour or returning segment of the tour as well as socioeconomic variables such as individual attributes have been included in the model.  For example, gender plays active and distinct roles in the model, increasing the utilities of private modes for males and increasing the utilities of taxi and transit for females.  In addition, some variables indicating job or workplace types have proved to be significant in the model.  However, few variables that indicate household attributes including income have been used in the model.  This may be because effects from those variables have been already captured indirectly by including the above-mentioned dummy that shows whether the same mode was used for the home-based main tour (Bradley et al., 1998).

#### Intermediate Stop Location Choice

Results of the estimated multinomial logit models are presented in Table 7.  Four different models have been estimated for work, school, maintenance, and discretionary main tour purposes.  The number of observations available for sampling differs by purpose; however, each model shows a fairly good fit with the adjusted rho-squared value ranging from 0.21 to 0.38.  Modeling outcomes are summarized and discussed below.

As a variable related to the cost and time of tours with an intermediate stop, extra generalized travel time to make the stop as compared to making no stop has been included.  In this way stops are more likely to be made in zones involving a shorter ‘detour’ in terms of generalized travel time.  This coefficient has been estimated as generic across the travel modes, and it has proved to be significant in all the four models.  Furthermore, variables indicating TOD in which a stop is made have been included in the models except for school tours.  Particularly in work tours, these variables imply that tours starting early in the morning reduce the utility of making a stop in the same zone as home or destination while tours with the second half (returning-home) departure in the night time increase the utility of making a stop in the same zone as home or destination.

For the intermediate stop location choice, some unique variables have been adopted in the models.  Variables indicating the geographical relationship between origin (home) and destination zones of the tour are among such variables.  In general, if a destination zone was selected from the zone group further from the origin in the home-based tour mode and destination choice model, it increases the utility of making a stop in a zone of the zone group further from both the origin and the destination, and vice versa.  Variables indicating the travel mode of the tour have also been included in the models, implying that most of the motorized travel modes except for motorcycle taxi are likely to increase the utilities of choosing zones further from the origin and destination zones.  To the contrary, motorcycle taxi and non-motorized transport are likely to increase the utilities of choosing a zone that is the same as the origin or the destination.

Among the zone-related variables included in the models, a dummy variable indicating whether the zone is inside the central business district (CBD) which is designated as the area for the “3-in-1” regulation has been newly included in the models except for discretionary tours.  This variable reduces the utilities of the CBD zones for stop location.  Some household-related variables have proved to be significant in the models; for example, people with children or infants in the household are more likely to make stops in the zones that are the same as the origin or destination (except for school tours) while higher-income people are more likely to make stops in the zones further from both the origin and the destination.  As for variables regarding individuals’ attributes, male adults tend to choose stops further from the origin and destination except for school tours.  Some variables indicating the status of individuals have also been included in the models, such as full-time workers (in work and discretionary tours), university students (in school and discretionary tours), and homemakers (in maintenance and discretionary tours).

## 4.2. Application to a Microsimulation model

All the models that have been estimated and described as above are the bases for a microsimulation model that generates activities/tours with full information including purpose, time of day, origin and destination zones, travel mode, and presence and details of sub-tours and intermediate stops for each household and individual based on the Monte Carlo simulation.  Those attributes of tours generated by the microsimulation are detailed enough to create OD trip tables by mode and by time of day.  In order to make the microsimulation more realistic, several important aspects of activity-based modeling should also be considered such as auto and motorcycle ownership choice, activity rescheduling, joint tour/activity generation, and household maintenance tour allocation.

### 4.2.1. Auto/Motorcycle Ownership Choice

Microsimulation for future years needs input about the number of autos and motorcycles owned by each household of different attributes.  In the Jakarta Metropolitan Area, 56 percent of households own neither autos nor motorcycles; meanwhile, 34 percent own motorcycle(s), and 17 percent own auto(s).  A household auto/motorcycle ownership model has been estimated with major combinations of numbers of autos and motorcycles owned by the household as choice alternatives.  It is a simple multinomial logit model, and the results are shown in Table 8.  Zone-related variables identifying the urban area and household density, variables indicating household composition, and a variable showing household income have proved to influence the auto/motorcycle ownership.  It is predicted that these variables, especially zone attributes and income information, will change over time, increasing the number of auto/motorcycle-owning households.

### 4.2.2. Activity Rescheduling

In the modeling system developed in this study, all activities/tours are first generated at the individual level.  Though both primary and secondary tours are simultaneously generated for each individual in the microsimulation, it is the primary tour that should have priority in the TOD choice.  Secondary tour(s) should then select the TODs for start of the tour and start of the returning segment of the tour from the remaining time windows of the day.  The TOD choice models developed in this study include variables indicating whether the tour is a secondary tour and especially whether it is attached to a work primary tour so that a combination of TODs would be most likely selected from the available time windows for the secondary tour.  However, it is not assured that the primary and secondary tours have no time conflict.  Therefore, if such a conflict has occurred, TODs of a secondary tour should be selected again based on a random draw to clear the conflict.

Moreover, sometimes a case occurs in which two or more secondary tours have been generated for an individual with a time conflict between them.  Secondary tours are classified into either maintenance or discretionary tours, and maintenance tours should be given priority in the TOD choice.  As such, secondary discretionary tours should be rescheduled if a time conflict occurs between these two activity purposes.  In the case of a conflict between secondary tours of the same purpose, one of the tours is given priority on a random basis and the other is rescheduled.

### 4.2.3. Joint Tour/Activity Generation

Intra-household interactions and decisions and its effect on daily activity-travel patterns should also be considered in the microsimulation.  In particular, the analysis of the joint tours/activities made by several members of the same household is an important facet of the activity-based transportation demand analysis and has been the subject of several studies in this area.  Most of the U.S. regional activity-based models take a further step and assert that travel behavior is not independent at the individual level but that there are linkages between household members.  For microsimulation development, several important rules have been adopted to generate joint tours/activities.

First, one of the basic findings, which has been replicated in several metropolitan areas, is that if a child stays home sick, this decreases the probability that adult household members will conduct their ‘typical’ mandatory travel for that day (Vovsha et al., 2004b).  As such, the following rule has been made.

•	If at least one of the school children of pre-driving age has a home (i.e., no travel) pattern, it is likely that at least one of the household adults has a home pattern to take care of the child.  In this case, each non-worker, part-time worker, full-time worker, or university student in the household has a different chance of having a home pattern at a certain probability depending on the status.  These probabilities should be applied to each household member having a mandatory activity.

Analysis of the activity diary survey (ADS) data has revealed several important characteristics of joint tours that seem to be unique to the Jakarta Metropolitan Area (Yagi and Mohammadian, 2005).  It was shown that contrary to what has been observed in the U.S. that travel for mandatory activities is assumed to have an individual character and usually joint mandatory activities are not included in activity scheduling models, mandatory activities in Jakarta have significantly higher joint tour ratio due to joint business and school activities.  Hence, the following two rules that are unique to Jakarta have been created in order to generate joint work and school tours, respectively.

•	If more than one workers in the household have the same job category and their primary activity is a work tour, it is likely that they work and travel together.  The probability that they have a joint primary work tour depends on the job category.  Different values of joint work tour probabilities should be applied to each case based on the job category.

•	If more than one school children belong to the same school type (elementary school, junior high school, senior high school, or university) and have a primary school tour, it is likely that they go to school and return home together.  The probability that they have a joint primary school tour depends on the school type.  Different values of joint school tour probabilities should be applied to each case based on the school type.

Furthermore, models developed for Atlanta and mid-Ohio explicitly model joint travel, which cannot be accounted for in conventional models (Scott and Kanaroglou, 2002; Vovsha et al., 2003).  Among others, the following two rules have been adopted into our microsimulation modeling system.

•	If at least one of the school children of pre-driving age has a non-mandatory (i.e., maintenance or discretionary) primary tour, it is often associated with visiting a doctor, family event, and so on.  In this case, each non-worker, part-time worker, full-time worker, or university student in the household has a chance of escorting the child and having the same primary non-mandatory tour at a certain probability depending on the status.  Adults having a higher chance of escorting the child are non-workers, part-time workers, full-time workers, and university students, in this order, and each adult household member should be tested with the probability derived from the observations.

•	If a full-time worker or a university student in the household does not have a primary work or school tour (i.e., having a day off) and chooses a non-mandatory (i.e., maintenance or discretionary) primary tour, it is often associated with major shopping, family event, vacation, and so on.  In this sense, it is likely that he or she is accompanied by at least one of the other adult household members.  In this case, each non-worker, part-time worker, full-time worker, or university student in the household has a chance of accompanying the worker/student and having the same primary non-mandatory tour at a certain probability depending on the status.  Accompanying adults are likely to be non-workers, part-time workers, full-time workers, and university students, in this order, and each adult household member should be tested with the probability derived from the observations.

### 4.2.4. Household Maintenance Tour Allocation

Another important feature of the intra-household interactions and decisions is a maintenance tour allocation in the household.  In the context of activity-based modeling, household maintenance activities, such as grocery shopping and escorting children, should be properly modeled at the household, rather than the individual level and then those should be allocated among household members (Vovsha et al., 2004a; Srinivasan and Bhat, 2004).  Though all activities are first generated at the individual level in the modeling system developed in this study, the following rule has been applied to the microsimulation to take the above concept into consideration.

•	Maximum number of maintenance tours per household for each household size is determined through the lookup table based on the 95th percentile values of the observed data.  If the number of simulated maintenance tours exceeds this maximum, maintenance tours are deleted from workers, students, homemakers, and others, whoever has exceeding number of maintenance tours.

# SUMMARY

This paper described a comprehensive multi-tier activity-based modeling system for Jakarta, consisting of random utility based disaggregate logit and nested logit models.  While the majority of explanatory variables have been directly derived from the ADS and HTS data, some other trip and zone-related data have also been defined such as generalized travel time computed from the highway and transit network, and land use composition in each TAZ computed from the GIS database.  Thus, a number of types of significant variables are included in the modeling system consisting of choices of daily activity-travel pattern, times of day, mode and destination, and intermediate stop location.  Interpretation of the effects of these explanatory variables in the developed modeled to several interesting insights.  Furthermore, it has been shown that, in the Jakarta Metropolitan Area or in the context of developing countries, some aspects of the modeling system including structure of the model, choice alternatives, and key variables are significantly different from those observed in the developed world.

For application of the estimated models to a microsimulation, further important aspects of activity-based modeling such as auto and motorcycle ownership choice, activity rescheduling, joint tour/activity generation, and household maintenance tour allocation have been considered.  Results from this activity-based microsimulation modeling system will be fed into a link-based transportation network assignment program in the form of OD trips by mode, and time of day.  Outputs from the network assignment model representing vehicle miles/hours of travel and average travel times can be used to evaluate various policy scenarios or economic impacts of different alternatives. Additionally, outputs by link such as volumes, average speeds, and average congestion levels will be used as part of the input data to the emission model.

Like many other metropolitan areas in developing countries, Jakarta is facing a transition from conventional infrastructure and highway construction to lower-cost but environment-friendly urban transportation management.  It is hoped that the new models developed in this study will contribute to a better understanding of urban travel behavior and improvement of the methodology of travel demand forecasting. It is also hoped that the study will provide a powerful decision-making tool that can be used to better analyze and evaluate urban transportation policy scenarios not only in the Jakarta Metropolitan Area but also in other urban areas of developing countries.

References

- Arentze, T.A., F. Hofman, C.H. Joh, and H.J.P. Timmermans (1998). Experiences with Developing ALBATROSS: A Learning-Based Transportation Oriented Simulation System. In: Verkehr und Mobilität: Stadt Region Land 66, (Edited by K.J. Beckermann), 61–70, Institut für Stadtbauwesen, Aachen.

- Axhausen, K. (1990). A Simultaneous Simulation of Activity Chains and Traffic Flow. In: Developments in Dynamic and Activity-Based Approaches to Travel Analysis, (Edited by P Jones), 206-225, Aldershot, Avebury.

- Bhat, C., S. Srinivasan, and J. Y. Guo (2003). Activity-Based Travel Demand Modeling for Metropolitan Areas in Texas: A Micro-Simulation Framework for Forecasting, Center for Transportation Research, FHWA/TX-03/4080-4, University of Texas.

- Bowman, J.L., M.A. Bradley, Y. Shiftan, K. Lawton, and M.E. Ben-Akiva (1998). Demonstration of an Activity Based Model System for Portland. Paper presented at the 8th World Conference on Transport Research, Antwerp, Belgium.

- Bowman, J.L., and M.E. Ben-Akiva (2000). Activity-Based Disaggregate Travel Demand Model System with Activity Schedules. Transportation Research Part A, 35, 1-28.

- Bradley, M.A., J.L. Bowman, and K. Lawton (1999). A Comparison of Sample Enumeration and Stochastic Microsimulation for Application of Tour-Based and Activity-Based Travel Demand Models. Paper presented at the European Transport Conference, Cambridge, U.K.

- Bradley, M.A., and P. Vovsha (2005). A Model for Joint Choice of Daily Activity Pattern Types of Household Members. Paper presented at the 84th Annual Meeting of the Transportation Research Board, Washington, D.C.

- Bradley, M.A., Portland Metro, J. L. Bowman, and Cambridge Systematics (1998). A System of Activity-Based Models for Portland, Oregon. USDOT Report DOT-T-99-02, produced for the Travel Model Improvement Program of the USDOT and EPA, Washington, D.C.

- Daganzo, C.F., and M. Kusnic (1993). Two Properties of the Nested Logit Model. Transportation Science, 27, 395-400.

- Ettema, D., A. Borgers, and H. Timmermans (1993). Simulation Model of Activity Scheduling Behavior. Transportation Research Record, 1413, 1-11.

- Golledge, R.G., M. P. Kwan, and T. Garling (1994). Computational-Process Modeling of Household Travel Decisions Using a Geographic Information System. Papers in Regional Science, 73(2), 99-117.

- Kitamura, R., and M. Kermanshah (1983). Sequential Model of Interdependent Activity and Destination Choices. Transportation Research Record, 944, 81-89.

- McFadden, D. (1978). Modeling the Choice of Residential Location. Transportation Research Record, 672, 72-77.

- Mohammadian, A., and S.T. Doherty (2005). Mixed Logit Model of Activity Scheduling Time Horizon Incorporating Spatial-Temporal Flexibility Variables. Transportation Research Record, 1926, 33-40.

- National Development Planning Agency (BAPPENAS), (2004). The Study on Integrated Transportation Master Plan for Jabodetabek (Phase 2), Final Report, prepared by Pacific Consultants International and ALMEC Corporation for the Republic of Indonesia and Japan International Cooperation Agency (JICA).

- Pendyala, R., R. Kitamura and A. Kikuchi (2004). FAMOS: The Florida Activity Mobility Simulator. Paper presented at the Conference on Progress in Activity-Based Analysis, May 28-31, 2004, Vaeshartelt Castle, Maastricht, The Netherlands.

- Recker, W. W., and M.G. McNally, (1986). An Activity-Based Modeling Framework for Transportation Policy Evaluation. In: Behavioral Research for Transportation Policy, 31-52. VNU Science Press, Utrecht.

- Salvini, P.A., and E.J. Miller (2005). ILUTE: An Operational Prototype of a Comprehensive Microsimulation Model of Urban Systems. Networks and Spatial Economics, 5, 217-234.

- Scott D., and P. Kanaroglou (2002). An Activity-Episode Generation Model that Captures Interaction between Household Heads: Development and Empirical Analysis, Transportation Research B, 36B, 875-896.

- Srinivasan, S., and C. Bhat (2004). Modeling the Generation and Allocation of Shopping Activities in a Household. Paper presented at the 83rd Annual Meeting of the Transportation Research Board, Washington, D.C.

- Van der Hoorn, T. (1983). Development of an Activity Model Using a One-Week Activity-Diary Data Base. In: Recent Advances in Travel Demand Analysis (Edited by S. Carpenter, and P. Jones), 335-349, Aldershot, Gower.

- Vovsha, P., E. Petersen, and R. Donnelly (2003). Explicit Modeling of Joint Travel by Household Members: Statistical Evidence and Applied Approach. Transportation Research Record 1831, 1-10.

- Vovsha, P., E. Petersen, and R. Donnelly (2004a). A Model for Allocation of Maintenance Activities to the Household Members. Paper presented at the 83rd Annual Meeting of the Transportation Research Board, Washington, D.C.

- Vovsha, P., E. Petersen, and R. Donnelly. (2004b). Impact of Intra-Household Interactions on Individual Daily Activity-Travel Patterns. Paper presented at the 83rd Annual Meeting of the Transportation Research Board, Washington, D.C.

- Yagi, S., and A. Mohammadian. (2005). An Exploratory Analysis of Intra-Household Joint Activity-Travel Tours in the Jakarta Metropolitan Area. Proceedings of the 6th Transportation Specialty Conference of the Canadian Society of Civil Engineers (CSCE), Toronto, Canada.

Table 1	Primary tour type classification

Table 2	Secondary tour type and number

Table 3	Daily Activity Pattern Choice Model (1/2)

Table 3	Daily Activity Pattern Choice Model (2/2)

Table 4	Home-Based Tour Time-of-Day Choice Model (1/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (2/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (3/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (4/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (5/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (6/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (7/8)

Table 4	Home-Based Tour Time-of-Day Choice Model (8/8)

Table 5	Home-Based Tour Mode/Destination Choice Model (1/4)

Table 5	Home-Based Tour Mode/Destination Choice Model (2/4)

Table 5	Home-Based Tour Mode/Destination Choice Model (3/4)

Table 5	Home-Based Tour Mode/Destination Choice Model (4/4)

Table 6	Work-Based Sub-Tour Mode/Destination Choice Model

Table 7	Home-Based Tour Stop Location Choice Model (1/2)

Table 7	Home-Based Tour Stop Location Choice Model (2/2)

Table 8	Household auto/motorcycle ownership model

Note: A is auto and M is motorcycle.

Figure 1	Overall modeling structure

Figure 2	Modeling structure: daily activity-travel pattern choice model

Note: Primary activities/tours are underlined and secondary tours are in italics.

Figure 3	Modeling structure: time-of-day choice model

Figure 4	Modeling structure: mode and destination choice model