Estimating the inter-urban French travel demand:

an econometric-based investigation.

Dr. Matthieu de Lapparent

Dr. Ariane Dupont

Abstract.

This article proposes an economic model of trip generation, distribution and mode choice for long distance travel. The approach combines disaggregated data and aggregated ones to produce estimations of passengers’ traffic flows and how they share among three modes of transportation in between employment areas in France. The results are along with intuition and the methodology gives the possibility to derive useful transportation indicators to understand long distance travel. In addition to the estimates of the model, it is given as examples the spatial distributions of the traffic elasticities by modes of transportation with respect to their own price attributes.

# I. Introduction.

Estimating the total mileage for long distance trips (over 100 kilometres on road network, or over 80 kilometres as the crow flies, as defined by the French National Institute of Statistics and Economic Studies - INSEE) and its repartition by types of vehicles or by modes of transport (main road network, railways, planes, excluding the maritime one) is a corner stone of the understanding of long distance mobility at the national level. This knowledge is particularly relevant for long term investment in transportation facilities as well as for transport policies (for instance computing elasticities of traffic with respect to travel time and travel cost to better understand the sensitivity of passengers’ flows with respect to the supply of transportation attributes). This approach aims at the computation of the optimal use of the transportation networks and the planning for building roads, railways and other kinds of transportation facilities. In a way, the main purpose is to understand the “optimal” use (as decided by the planner) of transportation networks by identifying and managing routes, cut routes, crossroads, congestion and safety on routes.

In this article we investigate a generation – distribution – mode choice model for passengers’ flows in between French metropolitan employment areas (a geographical clustering of France designed by INSEE). Freight flows are not explicitly modelled. We are considering at the same time the distribution of flows of travellers between the various geographic units and the share of these flows among three possible modes of transportation (road, rail, air) for each origin-destination (OD). The approach developed in this article differs from the classical aggregated four steps travel demand model (generation (G), distribution (D), mode choice (M), and traffic assignment (A)) in a given region, itself divided in several geographical zones. As a reminder (see Domencich and McFadden (1975) for an extensive presentation), generation deals with the number of trips produced and attracted by a geographical zone, distribution models how these trips spread between the considered geographical zones, mode choice deals with the splits of traffic flows in between the zones among different modes of transportation, and traffic assignment focuses on the circulation of these flows on the networks of the different modes of transportation. Although the GDMA approach has proven to give good results, it is mainly based on a mechanical approach that relies mostly on physical relations (for instance gravity equation, Abraham’s law, speed-flow relationships).

Furthermore, there are also nowadays several fully disaggregated travel demand forecasting systems (see for instance McFadden and Reid (1975), Ben-Akiva and Lerman (1985), who discuss necessary data and implementation of such models). The approach to be presented in this article lies in between a fully disaggregated model (Ben-Akiva and Lerman (1985))—that investigates the modal choice— and an aggregated one (Mandel, Gaudry, Rothengatter (1997))— that deals with the total flows between two zones and their socio-economic determinants). Our study focuses mainly on analyzing and explaining observed flows and how they share among three modes of transportation (road, rail, air), rather than attempting only to reproduce as well as possible these flows.

The rest of the article is organized as follows. Section 2 develops the model. The model is an attempt to estimate flows between two zones of employment and to explain them by socio-economic and transport factors. This modelling grounds on a twofold methodology. The flows analysis requires a prior understanding of the modal choice between two areas. Firstly, mode choices of travellers are then investigated at the microeconomic level. The econometric specification relies on random utility maximization and a linear multinomial Logit model. Secondly, the individual estimated behaviours are aggregated to analyze the passengers’ flows in between the considered employment areas. The econometric specification is then based on a log-linear regression model. Attention is paid in particular on the methodology used to define an unifying framework that is set up to analyze the roles played by socio-demographic and economic factors and transport attributes on traffic generation and its distribution among employment areas and modes of transportation. Section 3 details statistical data sources. They originate from various public administrations. The focus is made on the process by which data are compiled and harmonized to produce consistently the variables that are used to estimate the model. Section 4 discusses the results. The estimates from the discrete choice models and the estimates from the equation of passengers’ flows are interpreted. The outputs of the model enable many transportation indicators. It is given as example the aggregated rail elasticities with respect to rail fares: elasticities of each OD corridors are computed and then they are aggregated at the level of each employment area. In a last section are drawn conclusions and future research tracks are elaborated. Firstly, the scopes and the limits of the present approach are debated. Secondly, the emphasis is put on the role played by such a model in a broader approach designed to analyze road safety (TAG models, where TAG stands for Traffic, Accident, and Gravity) with a high level of accuracy.

# II. Model.

Consider that an origin (O) and a destination (D) are given. The flow of travellers in between these locations depends on several determining factors. Theory as well as observation lead to explain this flow by the sizes and the competitiveness of the different economic markets of these locations (job markets, tourist markets, recreational markets, markets of consumption goods, etc.), but also by the attributes of the transportation market that exists in between these two locations (variety of the modes of transportation, prices, capacities, frequencies, comfort, safety, etc.).

An individual who needs to travel from O to D faces mutually exclusive travel alternatives among which he has to choose one. They are described for the present application by generic modes of transportation (road, train, air), although they might be disaggregated more accurately. In the present approach, it is assumed that any traveller has always access to car mode, as either a driver or a passenger, and whatever his household does own a private motorized vehicle or not. It is also assumed that availability of rail and air modes of transportation is exogenously determined.

The total flow of travellers in between an origin employment area  and a destination employment area  by a mode of transportation  is defined as the total flow of travellers in between these two zones times the probability to choose that mode of transportation:

,

where the choice probability is defined as a weighted sum of choice probabilities for different types  of populations of travellers:

The weights  model the shares of individuals of type , among  possible types of population, who live in  and who travel in between  and . But due to the lack of data, these weights actually represent the shares of individuals of type  who live in . Another drawback due to the lack of data is that these weights will be computed at the level of the administrative geographical region that embeds the employment area .

As equation (1) shows, estimating the flows between two employment zones needs to model first modal choice between these two areas. In equation (2),  defines as the probability of an individual of type  to choose one of the modes of transportation that are available in between  and . These modes of transportation are described by attributes . The computation of these choice probabilities is based on the random utility paradigm (Block and Marshack (1960), McFadden (1973), McFadden (1978)). The probability that any traveller chooses a mode of transportation is equal to the probability that he maximizes his level of utility  for that mode of transportation. Considering that utility is the sum  of a deterministic part  and a random component  that is independently and identically distributed among the modes of transportation according to a Gumbel distribution, one obtains:

where the utility to choose a mode of transportation m by an individual of population type s, , is defined for the present application as

for the road mode in between  and ,

for the rail mode in between  and , and

for the air mode between  and . The variable “ovtime” stands for “out of vehicle time (access/egress time)” and “ivtime” variable stands for “in-vehicle time”. The latter variable accounts for the total time spend in the vehicle of the chosen mode of transportation.

It is important to highlight that there are several other functional forms that could have been chosen (or let be chosen by data as explained by Gaudry and Wills (1978) and implemented for instance in Mandel, Gaudry, and Rothengatter (1997)). However, the choice of the above functional forms accommodates very well to observed data. The linear form is tested in this paper as a reference function before further developments and improvements, especially in order to get closer to the theoretical definition of utility as a nonlinear decreasing function of transportation time and price attributes.

According to the decision rule that postulates that the traveller maximizes his utility level, and according to the theoretical definition of a utility function, only differences in utility levels matter. Because of the linear specification used in the present application, the coefficients that weight the traveller’s characteristics can not be estimated without further restrictions. A benchmark mode of transportation must be chosen to identify the estimable parameters. The coefficients are fixed to zero for it, so that the estimable coefficients express as differences of the impacts of travellers characteristics on choice probabilities with respect to that benchmark mode of transportation. Note also that utility is defined up to a monotone increasing transformation. Due to the linear structure of , it implies that all the parameters are scaled by . From an empirical point of view, it means that the numerical values of the estimated parameters have no interpretation: only matter their signs and their statistical significance.

The total flow of travellers  in between  and  is defined as

.

is a scale factor that groups together in the present application different economic and demographic characteristics of the employment areas  and  (GNP per capita, job density, etc.). It is defined as the product of different explanatory variables:

is an index of efficiency of the transportation market that is defined for the OD corridor in between  and . as

.

It models the expectation of the maximum of utility  of travelling in between  and .

# III. Data.

In the present approach, the focus is on trips and total passenger flows in between 341 employment areas within the metropolitan French territory. Maps presented in the appendix give a picture of the design of the employment areas. Corsica as well as the other small islands near to the maritime coasts are excluded from the analysis. This selection is made because of a lack of data about the levels of services of the maritime mode of transportation.

Several data sources are used and gathered to prepare the necessary data to estimate the mode choice models and the equation of passenger traffic flows. The primary data source used for the estimation of the mode choice models is the 1993 French National Household Travel Survey (ENT93). Three data files are used to describe how and what influenced the choices travellers made for long distance trips: the file that reports long distance trips (origin and destination locations, modes of transportation, travel times, travel distances, and trip purposes), the file that reports the characteristics of the travellers (age, gender, driving license), and the file that reports the characteristics of the households of the travellers (car ownership). The ENT93 data are also used to compute the shares of 48 types of travellers: these types are obtained by making the Cartesian product of three age classes (less than 20 years old, between 20 years old and 65 years old, more than 65 years old), two gender classes (male or female), two classes for driving license (yes or no), two trip purposes (personal or professional), and two classes for car ownership (owning at least one private motorized vehicle or not). These shares are calculated for each of the 22 administrative regions of France. It is not possible to compute these shares at a more disaggregated geographical level without loosing the representativeness of the sample for the whole French population.

In addition to the ENT93 data, several other secondary sources were used to build the travel alternatives the travellers faced before observing their chosen modes of transportation, but also to build the necessary data to estimate the equation of passenger traffic flows. Data provided by the economic unit (SESP) of the French Ministry of Transport, Housing and Facilities are used to facilitate the investigation. Data come from the MODEV model (multimodal model of inland transport of goods and travellers for long distances, see the reports of Surineau and Melin (2001)), which is based on a classical four steps modelling approach. Input data and transportation networks come from different French administrations and research units. The road network comes from the French Roads and Highways Technical Studies Unit (SETRA) and it depicts highways and national roads as well as few departmental roads. The rail network comes from the French National Institute of Research on Transport and Safety (INRETS). It is based on connections in between 229 train stations and it contains connections made by classical trains as well as high speed trains. The air network comes from the National Administration of Civil Aviation (DGAC). It is based on 116 connections in between 44 airports. Average daily flows as well as supply characteristics (capacities, frequencies, free flow speeds, etc.) are provided for each of these networks. At the end of the traffic assignment procedure (shortest paths for the rail mode and the air mode, and Wardrop (1952)’s user equilibrium for the road mode), the MODEV model gives as outputs the levels of services (LOS) on the different origin-destination corridors. For all the modes of transportation, travel times, travel distance, and travel budget costs, are available. Travel price for the road mode is computed as the product of travel distance with a unit kilometre cost plus toll costs if there are ones. Travel prices for the air mode are computed as the sum of the cost to access and egress from airports by using the road mode (the choice of access and egress modes are not modelled) and the ticket fare (as given by the SESP). Travel prices for the rail mode are computed as the sum of the cost to access and egress from the train stations (the choice of access/egress modes are not modelled) by using the road mode and the ticket fare (as given by the SESP). Travel times for the rail mode and the air mode are distinguished according to travel times to access/egress from the airports/train stations by road and travel times on the rail network/air network. Note that using these levels of services defined for pairs of employment areas (from centroid to centroid) introduces measurement errors on the travel attributes the travellers are facing for their trips, which are reported at a more disaggregated geographical level, as to say for trips from cities to cities.

For the present application, the average daily counts on road network are converted from vehicle counts to passenger counts by using vehicle occupancy rates calculated by the SETRA. The total passenger flows in between employment areas are obtained by summing observed passenger flows over the different available modes of transportation. Finally, these statistical data are complemented with census data about the employment areas provided by the French National Institute of Statistics and Economic Studies (INSEE). All these data were grouped together to have available a set of variables to estimate the equation of passengers traffic flows. The maps (map 1 to map 5) presented in the appendix give a picture of the variables used for the estimation of the equation of passenger flows.

One discrete choice model is estimated over the whole sample of travellers being assumed that available modes of transportation are given exogenously for all the possible OD pairs. Once estimated the parameters of the utility functions and the weights of the different types of travellers, the choice probabilities as well as the logsum variables are aggregated at the level of the employment areas. These latter were then used as explanatory variables in the equation of traffic flows. Note that the logsum is equal to 0.85 that fits to the idea that the improvement in transportation conditions implies an increase in travel demand.

# IV. Results.

Table 1 reports maximum likelihood estimates of the mode choice model. The identification of estimable parameters is made by choosing a benchmark mode of transportation, a benchmark individual type, and a benchmark trip purpose. The mode of transportation “road” is the benchmark mode for the identification of the parameters that enter the utility of travellers. The type of individual of reference is chosen to be a male between 20 years old and 65 years old without driving license and who lives in a household who owns at least one private motorized mode of transportation. The benchmark trip purpose is a personal one (as opposed to a professional trip).

The results show that prices and times have negative effects on choice probabilities. Whatever the choice set is, the probability to choose one of the available modes of transportation decreases as long as its price and/or its time increase.

An attribute that is specific to the air mode is the frequency of flights originating from the airport the traveller can access for his trip. As expected, the positive sign of the associated coefficient and its statistical significance show that the more the origin airport offers flights, the more it is attractive and the more any traveller tends to consider this travel alternative.

One interesting result is that the gender of the traveller does not play a significant role on the choice of a mode of transportation. One could have thought that male travellers and female travellers have different lifestyles as well as different habits and therefore they do not make the same choices. But the result for the present application shows from a statistical point of view that the choice probabilities do not differ.

As regards to professional trips, the probability to choose a collective mode of transportation is larger than the probability to choose a collective mode of transportation for personal purposes. It could be explained by the fact that the employer covers better the travel expenditures for rail and air modes than for the road mode. It could also be explained by the fact that the traveller has a more limited mobility at destination and therefore he does not necessarily need a car.

Travellers who are between 20 years old and 65 years old have a larger probability to choose the road mode than travellers who are over 65 years old or who are under 20 years old. For travellers who are over 65 years old, the main explanation has to be sought towards the probably larger level of fatigue induced by long distances to drive. For young travellers, the main explanation has to be sought towards the greater probability of non availability of a private motorized vehicle and/or the non possession of a driving license.

Travellers who own a driving license tend to choose the road mode with a higher probability than travellers who do not own one. The result is not hardly surprising to the extent that there is no need to rely on someone else to use the road mode.

Not surprisingly, a traveller who lives in a household without a private motorized vehicle has a larger probability to choose the rail mode of transportation, or the air mode of transportation when available, than a traveller who lives in a household with at least one private motorized vehicle.

Table 2 reports quasi-generalized least squares estimates of the equation of traffic flows. Because of the log-linear structure of the flow equation, the estimates are interpreted as elasticities of the traffic flow with respect to the variables they weight. As expected, the coefficients have the right signs and they do not exhibit excessive values that could show a wrong specification of the model.

GNP per capita plays an important (and statistically significant) role on the generation and the distribution of travellers’ flows to and from an employment area. It is a result along with intuition. The population densities at origin and at destination play also a positive role on the generation and the distribution of traffic flows from an employment area to the other employment areas. Among the different elasticities, one observes that the GNP per capita plays a prevailing role. The second utmost effect on the generation and the distribution of travel flows is population density. It fits to the usual results of generation-distribution and gravity models: the number of inhabitants and their levels of wealth matter. Indeed, a large population density implies the development of various and numerous activities (public facilities, health centres, commercial activities, cultural activities, industrial activities, etc.), which in turn attract people who do not live in the area of interest. Furthermore, it attracts people who are already living in dense areas.

There are additional factors that explain the generation and the distribution of traffic flows. They are connected to the economic growth potential of the employment areas. For instance, the tourist production capacities at origin and at destination play a positive role on the generation of the traffic flows. The size of the tourism market is a significant factor that affects the level of attraction and the level of production of traffic flows.

But the job density has a negative impact on flows. The increase of the job market in one area reduces the level of long distance flows between i and j: people tends to prefer to live near from their workplaces (within a radius lower than 100 km) and therefore try to avoid to travel over 100 km to reach them. Given an employment area, a higher job density lends to settle its inhabitants and those from its neighbourhood.

Several tests of restriction on the parameters of the model conclude that the coefficients associated to the socioeconomic and the demographic characteristics of both the origin zone and the destination zone are not statistically different. Thereby, the parameters associated to the tourist capacities at origin and at destination are not significantly different and the parameters associated to the GNP per capita at origin and at destination are not significantly different. The parameters associated to the population densities at origin and at destination are also not significantly different, and finally the parameters associated to the job densities at origin and at destination are not significantly different.

The logsum parameter gives a measure of the impact of the efficiency of the transportation market in between an origin employment area and a destination employment area on the level of traffic flow. As expected, it increases as long as the supply of transport provides low travel times and/or low travel costs. The positive sign of the logsum parameter and the negative signs associated to both price and time coefficients that enter the utility functions show that the total demand for transportation decreases as price and/or time increase. It is also observed for all the modes of transportation that travel time plays a more important role than travel price: the traffic flows are more elastic with respect to travel time as compared to travel price. For long distance trips, time is the paramount component of the generalized cost of travel.

It is possible to aggregate more the results to obtain useful transportation indicators. For instance, maps 6, 7,8, depict the spatial distributions of the weighted average elasticities of traffic emissions of employment areas for the three modes of transportation with respect to their own price attributes. Actually, elasticities are computed for each of the OD pairs, then they are weighted by their rates of contribution to the total traffic emission (total traffic starting from O), and they are finally summed by employment areas.

# V. Conclusions.

In addition to several results of interest, one of the main contributions of the present approach is propose a methodology that mixes disaggregated data (traveller’s choices) and geographically aggregated ones (traffic flows). But there are several limitations to the present approach. Firstly, the model is static and based on cross-sectional data. Secondly, the model does not deal with traffic assignment on transportation networks as well as many other choices that rule the observed flows in between the employment areas (for instance destination choices and route choices). Thirdly, the econometric specification is based on a multinomial Logit model with a linear representation of behaviours and a log-linear representation of traffic flows. It is also important to note that the present model belongs to a broader and complex modelling approach, which attempts to estimate the total mileage for France on different transportation networks. The model proposed in this article is a first step towards an integrated model designed to analyze road safety with a high level of accuracy. By estimating traffic on different networks, different purposes, and different classes of distance, it should be possible to depict more accurately the exposure to risk of accident.

# References.

- Andrikopoulos, A.A, and J.A. Brox (1990), Canadian Inter-City Passenger Transportation: a Simultaneous Equation Approach, International Journal of Transport Economics, Vol. 17, pp. 311-328.

- Bel I Queralt, G (1995), Interurban Competition on Inter-Urban Rail, International Journal of Transport Economics, Vol.17, pp. 181-198.

- Ben-Akiva, M., and S. Lerman (1985), Discrete choice analysis: theory and application to travel demand, MIT Press.

- Block, H., and J. Marshack (1960), Random orderings and stochastic theories of response, in I. Olkin, ed., Contributions, to probability and statistics, Stanford University Press.

- Calio J, and J.C. Méteyer (2001), Le modèle multimodal de transport intérieur à longue distance du SES : MODEV, Note de synthèse du SESP, 8 pages.

- Coto-Millan, P. (1996), Intermodal Competition on Inter-Urban Rail: Theoretical and Empirical Microfoundations, International Journal of Transport Economics, Vol.23, pp. 379-381.

- Domencich, T., and D. McFadden (1975), Urban travel demand : a behavioral analysis, North Holland Publishing Company, Amsterdam.

- Gaudry, M., and S. Lassarre (2000), Structural Road Accident Models: the International DRAG family, Pergamon Elsevier, 347 pages.

- Gaudry, M., and M. Wills (1978), Estimating the functional form of travel demand models, Transportation Research B, Vol.12, pp.257-289.

- INSEE (1999), Official Geographic Zones and Census data, http://www.insee.fr

- Laferrière R. (1993), Les élasticités-prix de la demande de transport interurbain des personnes, Les Cahiers Scientifiques du Transport, Vol. 27, pp. 37-70.

- Mandel, B., Gaudry, M., and W. Rothengatter, 1997, A Disaggregate Box-Cox Logit Mode Choice of Intercity Passenger Travel in Germany and its Implications for High Speed Rail Demand Forecasts, The Annals of Regional Science, Vol.31, pp.99-120.

- McFadden, D. (1973), Conditional Logit Analysis of Qualitative Choice Behavior, in P. Zarembka, ed., Frontiers in Econometrics, pp.105-142, Academic Press: New York

- McFadden, D., and F. Reid (1975), Aggregate Travel Demand Forecasting from Disaggregated Behavioral Models, Transportation Research Records, No.534, pp.24-37.

- McFadden, D. (1978), Modelling the Choice of Residential Location, in A. Karlqvist, L. Lundqvist, F. Snickars, and J. Weibull, eds., Spatial Interaction Theory and Planning Models, pp.75-96, North Holland: Amsterdam.

- Rickard JM. (1988), Factors Influencing Long Distance Rail Passenger Trip Rates in Great Britain, Journal of Transport Economics and Policy, Vol.22, pp. 209-234.

- SETRA,  http://www.setra.equipement.gouv.fr

- Surineau, A., and S. Melin (2001), MODEV user guide, report prepared for the French Ministry of Transportation, Housing, and Facilities, Economic and Prospective Studies Unit.

- Surineau, A., and S. Melin (2001), MODEV methodoly, report prepared for the French Ministry of Transportation, Housing, and Facilities, Economic and Prospective Studies Unit.

- Wardman, M. (1997a), Inter-Urban Rail Demand Elasticities and Competition in Great Britain: Evidence From Direct Demand Models, Transportation Research Part E, Vol. 33, pp. 15-28.

- Wardman, M., Toner J.P., and G.A. Whelan (1997b), Interactions Between Rail and Car in the Inter-Urban Leisure Travel Market in Great Britain, Journal of Transport Economics and Policy, Vol.31, pp. 163-181.

- Wardrop, J. (1952), Some theoretical aspects of road traffic research, Proceedings of the Institute of Civil Engineers, part II, pp.325-378, London.

# Appendix.

## Tables.

| Variables | ### Value | t-stat |
| --- | --- | --- |
| Rail mode intercept | -2.1008 | -16.1021 |
| Air mode intercept | -7.5357 | -13.0027 |
| Dummy: professional purpose (air mode) | 2.4239 | 22.4740 |
| Dummy: professional purpose (rail mode) | 1.6674 | 34.7011 |
| Dummy: age under 20 y.o. (air mode) | -0.0763 | -0.1348 |
| Dummy: age under 20 y.o. (rail mode) | 1.2857 | 10.1482 |
| Dummy: age over 65 y.o. (air mode) | -0.2108 | -1.1928 |
| Dummy: age over 65 y.o. (rail mode) | 0.2228 | 3.7129 |
| Dummy: female gender (air mode) | 0.1308 | 1.2179 |
| Dummy: female gender (rail mode) | 0.0189 | 0.4280 |
| Dummy: traveller with a driving license (air mode) | -0.2275 | -0.9218 |
| Dummy: traveller with a driving license (rail mode) | -0.3922 | -5.0073 |
| Dummy: household without private motorized vehicle (air mode) | 0.6798 | 3.3919 |
| Dummy: household without private motorized vehicle (rail mode) | 1.9246 | 28.1381 |
| Log of flight frequency originating from departure airport | 0.9069 | 17.2627 |
| Trip price in euros | -0.0163 | -25.3551 |
| Log of trip in vehicle time in minutes | -1.4489 | -23.6294 |
| Log of access time in minutes | -0.4120 | -13.6289 |
| Goodness of fit |  |  |
| Sample size | 24988 |  |
| Log-likelihood ratio | 22716.1 (16 df) |  |
| Pseudo R-square | 0.5329 |  |

Table 1: Multinomial Logit choice model

| Variables | ### Value | t-stat |
| --- | --- | --- |
| Intercept | -9.1650 | -21.05 |
| Log of population density at origin | 0.5205 | 24.75 |
| Log of population density at destination | 0.5326 | 25.33 |
| Log of job density at origin | -0.0954 | -4.49 |
| Log of job density at destination | -0.0961 | -4.52 |
| Log of net income before taxes per inhabitant at origin | 0.7880 | 27.34 |
| Log of net income before taxes per inhabitant at destination | 0.7905 | 27.42 |
| Log of tourist density at origin | 0.4289 | 143.72 |
| Log of tourist density at destination | 0.4227 | 141.82 |
| Log of logsum variable | 0.8518 | 344.29 |
| Goodness of fit |  |  |
| Sample size | 106540 |  |
| Residual variance | 0.9779 |  |
| Fisher’s test | 26742.2 (9 df) |  |
| R-square | 0.6932 |  |

Table 2: equation of passengers' flows, quasi-generalized least squares estimates

## Maps.

Map 1: net income before taxes per inhabitant

Map 2: job density

Map 3: population density

Map 4: tourist density: camping places

Map 5: tourist density: hotel rooms

Map 6: elasticity of road traffic emission of employment areas with respect to road price

Map 7: elasticity of rail traffic emission of employment areas with respect to rail price

Map 8: elasticity of air traffic emission of employment areas with respect to air price

 Corresponding author. French National Institute of Research on Transport and Safety - INRETS.Department of Transport Economics and Sociology – DEST. 2, avenue du Général Malleret-Joinville. 94114, Arcueil, France. Tel. : +33(1) 47 40 72 63. E-mail : lapparent@inrets.fr

 French National Institute of Research on Transport and Safety, INRETS. Department of Transport Economics and Sociology, DEST. 2, avenue du Général Malleret-Joinville. 94114, Arcueil, France. Tel. : +33(1) 47 40 72 73. E-mail : ariane.dupont@inrets.fr

The reader should keep in mind that it is not the case for all the models that rely on the GDMA approach. There are today several other types of models that keep the four step structure but that introduce more behavioural and probabilistic assumptions about how traffic is generated and distributed among geographical zones.

Note that for an infinite number of types of population S, the model would be asymptotically equivalent to a fully disaggregated model.

The Gumbel distribution has cumulated density function defined as . It is also assumed in the present approach that .